Please read the manual carefully before installing and using the product

User Manual

824+A665+A589-4-color waterproof lamp

directory

1. Precautions and installation precautions and installation	3

1.1 Statement	3

1.2 Maintenance	3

1.3 Product precautions	3

1.4 Product introduction	4

2. Display panel and key definitions	4

2.1 Control panel	4

2.2 Menu function	4

3. Master-slave control	6

4. DMX512 control	6

4.1 Address Code	6

4.2 CH03, CH04, CH05, CH06, AH06 channels	6

4.3 CH09, CH13, CH19, RGB12CH, RGB17CH channels	7

4.4 CMY12CH. CMY17CH channel	7

4.5 CH15, CH28 channels	8

5. Frequently Asked Questions	11

1. Precautions and installation precautions and installation

1.1 Statement

Thank you for choosing our products! This product leaves the factory with good performance and complete packaging. For you to use this product safely and effectively, please read this instruction manual carefully and completely before using this product. This manual contains important information for installation and use, please follow the requirements of the manual for installation and operation, and at the same time, please keep this manual properly for use at any time.

This manual is subject to technical changes without notice.

1.2 Maintenance

- This fixture is intended for professional use only.

- If the external flexible cable or wire of this luminaire is damaged, it should be replaced with a special wire or wire provided by the manufacturer or its service agent.

- The light source contained in this luminaire may only be replaced by the manufacturer or its service agent or similar qualified personnel.

- Always connect the product to a protected circuit (circuit breaker or fuse). Ensure that the product has proper electrical grounding to avoid the risk of electric shock or fire.

- To avoid unnecessary wear and extend its life, when not in use, the power supply should be completely cut off by circuit breaker or unplugged.

- The housing may heat up while the product is running. This product should be installed in a well-ventilated location.

- When the product is transferred from an extreme temperature environment (e.g., from a cold truck to a warm and humid environment), the internal electronics of the product can condense. To avoid malfunctions, allow the product to adapt to its surroundings before turning on the power.

- Flash is known to trigger seizures. Users must comply with local notification laws regarding flash use.

1.3 Product precautions

- Disconnect the power supply before cleaning the product or replacing the fuse.

- When not in use, replace and secure all power, data, USB, or other port shields to IP protection.

- Use a safety rope when attaching this product above your head.

- Connect this product to a grounded and protected circuit.

- When the product is on, look at the light source.

- Do not place any flammable materials within 20  cm of this product when operating or turning on the power  supply.

- Do not operate this product if the housing, lenses, or cables are damaged.

- This product can only be handled using handles or hoisting/mounting brackets.

- To avoid unnecessary wear and extend its life, when not in use, the power supply should be completely cut off by circuit breaker or unplugged.

- In case of serious operating problems, it should be stopped immediately

1.4 Product introduction

- Voltage:  AC100~240V  50/60HZ

- Power: 148W

- Bulb:  COB-1 RGBW (120W) + Auxiliary light 32 pieces 5050-RGB

- Control mode: DMX512, self-propelled, master-slave, voice control, with RDM function.

- Channel: CH03, CH04, CH05, CH06, AH06, CH09, CH13, CH19,

RGB12CH,RGB17CH,CMY12CH,CMY17CH,CH15,CH28

- Dimming: 32bit 0~100% linear dimming

- Features: dyeing + flashing + auxiliary light

- Operating temperature: -30°C~60°C

- Strobe frequency: 1~30HZ

- Appearance: Metallic, black

- Connection: DMX512 input/output / power input/output.

- IP Rating: IP67

- Size:

- Weight:

2. Display panel and key definitions

2.1 Control panel

| Menu key: enter/return to the main menu Up key: The menu function selects up/adds parameters Down key: Menu function selects down/decrements parameters Confirm key: Select the menu function, and confirm and save it when the menu function parameters are modified. | Steps to unlock the screen: (1) Long press the button corresponding to ESC yellow; (2) Press UP; (3) Press DOWN; (4) Press ENTER. If the display corresponds to the correct touch button, it will automatically jump to the next one *The buttons remain unchanged when inverted, and you still follow the instructions above |
| --- | --- |

2.2 Menu function

After powering on and resetting the whole machine, press the menu button to enter the main menu (exit after 30 seconds of no operation on the main menu interface), see the main menu menu (a total of 5 main menu options); Select the main menu with the up or down button, click the confirmation button to modify the function parameters of each menu, and then press the confirmation button to save the current function parameters (with power loss memory after saving).

| Main menu | Submenu | Menu function description |
| --- | --- | --- |
| 1. System settings | 1. DMX address | Modify the DMX address code up or down (the address code can be selected in the range 001-512), and confirm that the key is saved, default 001. |
|  | 2. Signal retention | Select Off/On settings up or down, confirm that the key is saved, and it is turned off by default. |
|  | 3. Key lock | Select Off/On settings up or down, confirm to save, and turn on by default. |
| 2. Operation mode | 1. Operation mode | Select the running mode up or down, DMX/Voice Control/Auto/Manual, confirm the key to save, default DMX. |
|  | 2. Channel selection | Select channels CH03, CH04, CH05, CH06, AH06, CH09, CH13, CH19, RGB12CH, RGB17CH, CMY12CH, CMY17CH, CH15, CH28 confirmation key to save, default CH28. |
| 3. Manual control | 1. Focusing | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 255. |
|  | 2. Dimming | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 255. |
|  | 3. Dimming fine-tuning | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 255. |
|  | 4. Strobe | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 5. Red | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 6. Red fine-tuning | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 7. Green | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 8. Green fine-tuning | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 9. Blue | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 10. Blue fine-tuning | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 11. White | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 12. White fine-tuning | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 13. Color temperature | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 14. Macro color | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 15. Pattern | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 16. Speed | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 17. F dimming | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 18. F strobe | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 19. F red | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 20. F green | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 21. F blue | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 22. F mode | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 23. F speed | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 24. F background color | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 25. F background color dimming | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
|  | 26. Reset | Modify the parameter value up or down (adjustable range 000-255), confirm that the key is saved, default 000. |
| 4. Display settings | 1. Language selection | Select Chinese/EN (English) up or down, confirm the key to save, default Chinese. |
|  | 2. Inverted settings | Select Turn off/on the display setting up or down, confirm that the key is saved, and turn off by default. (The keys remain unchanged) |
|  | 3. Screensaver settings | Select Turn off/on screensaver settings up or down, confirm the key to save, and turn on by default. |
|  | 4. Software version | G8RA program identification code, cannot be modified. |
| 5. Factory setting | 1. Focus calibration | Modify the parameter (000-255) up or down, make sure the key is saved, and the default is 128. |
|  | 2. Red | Modify the parameter (032-255) up or down, and make sure the key is saved, default 255. |
|  | 3. Green | Modify the parameter (032-255) up or down, and make sure the key is saved, default 255. |
|  | 4. Blue | Modify the parameter (032-255) up or down, and make sure the key is saved, default 255. |
|  | 5. White | Modify the parameter (032-255) up or down, and make sure the key is saved, default 255. |
|  | 6. F red | Modify the parameter (032-255) up or down, and make sure the key is saved, default 255. |
|  | 7. F green | Modify the parameter (032-255) up or down, and make sure the key is saved, default 255. |
|  | 8. F blue | Modify the parameter (032-255) up or down, and make sure the key is saved, default 255. |
|  | 9、LED | Select LEDA/LEDB up or down, confirm that the key is saved, and the default LEDA is the default. |
|  | 10、Logo | Select 000/001 settings up or down, confirm that the key is saved, default 000. |
|  | 11. Temperature | Modify the parameters up or down (040-070°C) and make sure the key is saved, default 065. |
|  | 12. Temperature-fan | Select the on/off setting up or down, confirm that the key is saved, and off by default. |
|  | 13. Dimming curve | Select Linear/Squared/Inverse/S-Curve/up or down, and confirm that the key is saved, and the default linearity is true. |
|  | 14. Dimming mode | Select Stage/TV/Architecture/Theater/Stage 2/Standard up or down, confirm the key to save, default linear. |
|  | 15. Dimming frequency | Select 1KHZ/2KHZ/4KHZ/7KHZ/9KHZ/11KHZ/14KHZ/16KHZ/18KHZ/21KHZ/, and confirm that the key is saved, the default is 11KHZ. |
|  | 16、ETO | Modify the parameter (000-255) up or down, make sure the key is saved, default 255. |

3. Master-slave control

There are 2 or more identical lamps connected with DMX three-core signal cables, all lamps are modified to 001~512 arbitrary address code, any one is set as the host, other lamps are slaves, and all slave displays do not flicker; When the main unit is used to adjust the gradient, pulse change, jump, voice control, and self-propelled effects, all slave synchronized gradients, pulse changes, jumps, voice-controlled, and auto-propelled effects.

Special attention: 1. A group of lamps can only set one host, if there are multiple hosts, all the lamps will flash randomly and not synchronized.

- All luminaires must be active when the DMX512 console is turned off.

4. DMX512 control

4.1 Address Code

All the lamps are set with the address code, connect all the lamps to the DMX512 console in parallel with DMX three-core signal cables, the address code stops flashing, indicating that the DMX512 console signal has been sent to the lamp, and use the DMX512 console to control the relevant functions according to the channel instructions.

4.2 CH03, CH04, CH05, CH06, AH06 channels

| CH03 | CH04 | CH05 | CH06 | AH06 | Channel name | DMX value |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | Focusing | 0-255 |
| 2 | 2 | 2 | 2 |  | Dimming | 0-255 |
|  |  | 3 |  |  | Strobe | 0-255 |
|  | 3 | 4 |  |  | Color temperature preset | 0-255 |
|  |  |  | 3 | 2 | red | 0-255 |
|  |  |  | 4 | 3 | green | 0-255 |
|  |  |  | 5 | 4 | blue | 0-255 |
|  |  |  |  | 5 | white | 0-255 |
| 3 | 4 | 5 | 6 | 6 | reposition | 0-249 is invalid 250-255 Reset |

4.3 CH09, CH13, CH19, RGB12CH, RGB17CH channels

| CH09 | CH13 | CH19 | RGB12CH | RGB17CH | Channel name | DMX value |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | Focusing | 0-255 |
| 2 | 2 | 2 | 2 | 2 | Dimming | 0-255 |
| 3 | 3 | 3 | 3 | 3 | Dimming fine-tuning | 0-255 |
| 4 | 4 | 4 | 4 | 4 | Strobe | Shutter |
|  | 5 | 5 | 5 | 5 | red | 0-255 |
|  |  | 6 |  | 6 | Red fine-tuning | 0-255 |
|  | 6 | 7 | 6 | 7 | green | 0-255 |
|  |  | 8 |  | 8 | Green fine-tuning | 0-255 |
|  | 7 | 9 | 7 | 9 | blue | 0-255 |
|  |  | 10 |  | 10 | Blue fine-tuning | 0-255 |
|  | 8 | 11 |  |  | white | 0-255 |
|  |  | 12 |  |  | White fine-tuned | 0-255 |
| 5 | 9 | 13 | 8 | 11 | Temperature | 0-9 is invalid 10-255 One value, one effect |
|  |  | 14 |  | 12 | Color temperature fine-tuned | 0-255 |
| 6 | 10 | 15 | 9 | 13 | Red-green ratio | 0-255 (color temperature takes effect) |
| 7 | 11 | 16 | 10 | 14 | Macro color | Macro color |
|  |  | 17 |  | 15 | Dimming control | Dim Control |
| 8 | 12 | 18 | 11 | 16 | control | Control |
| 9 | 13 | 19 | 12 | 17 | reposition | 0-249 is invalid 250-255 Reset |

4.4 CMY12CH. CMY17CH channel

| CMY12CH | CMY17CH | Channel name | DMX value |
| --- | --- | --- | --- |
| 1 | 1 | Focusing | 0-255 |
| 2 | 2 | Dimming | 0-255 |
| 3 | 3 | Dimming fine-tuning | 0-255 |
| 4 | 4 | Strobe | Shutter |
| 5 | 5 | cyan | 0-255 |
|  | 6 | Fine-tuned in cyan | 0-255 |
| 6 | 7 | magenta | 0-255 |
|  | 8 | Magenta fine-tuning | 0-255 |
| 7 | 9 | Yellow | 0-255 |
|  | 10 | Yellow fine-tuned | 0-255 |
| 8 | 11 | Temperature | 0-9 is invalid 10-255 One value, one effect |
|  | 12 | Color temperature fine-tuned | 0-255 |
| 9 | 13 | Red-green ratio | 0-255 (color temperature takes effect) |
| 10 | 14 | Macro color | Macro color |
|  | 15 | Dimming control | Dim Control |
| 11 | 16 | control | Control |
| 12 | 17 | reposition | 0-249 is invalid 250-255 Reset |

4.5 CH15, CH28 channels

| CH15 | CH28 | Channel name | DMX value |
| --- | --- | --- | --- |
| 1 | 1 | Focusing | 0-255 |
| 2 | 2 | Dimming | 0-255 |
| 3 | 3 | Dimming fine-tuning | 0-255 |
|  | 4 | Strobe | Shutter |
| 4 | 5 | red | 0-255 |
| 5 | 6 | Red fine-tuning | 0-255 |
| 6 | 7 | green | 0-255 |
| 7 | 8 | Green fine-tuning | 0-255 |
| 8 | 9 | blue | 0-255 |
| 9 | 10 | Blue fine-tuning | 0-255 |
| 10 | 11 | white | 0-255 |
| 11 | 12 | White fine-tuned | 0-255 |
|  | 13 | Temperature | 0-9 is invalid 10-255 One value, one effect |
|  | 14 | Macro color | Macro color |
|  | 15 | mode | 0-51 is invalid 52-103 Mode 2 104-155 Mode 3 156-207 Mode 4 208-255 Mode 5 |
|  | 16 | velocity | 0-255 |
|  | 17 | Supplementary light dimming | 0-255 |
|  | 18 | auxiliary light strobe | Shutter |
| 12 | 19 | Fu Guang R | 0-255 |
| 13 | 20 | Supplementary light G | 0-255 |
| 14 | 21 | Auxiliary light B | 0-255 |
|  | 22 | Supplementary light mode | 0-2 is invalid 3-254 Mode 1 - Mode 84 (3 values and one effect) 255 Mode 85 (All Effects Loop) |
|  | 23 | Assist light speed | 0-255 |
|  | 24 | Assistive background color | Background color |
|  | 25 | Supplementary background dimming | 0-255 |
|  | 26 | Dimming control | Dim Control |
|  | 27 | control | Control |
| 15 | 28 | reposition | 0-249 is invalid 250-255 Reset |

Shutter

| serial number | Channel value | Basic features: |
| --- | --- | --- |
| 1 | 0-31 | The lamp beads went out |
| 2 | 32-63 | Always bright |
| 3 | 64-95 | Normal strobe |
| 4 | 96-127 | Always bright |
| 5 | 128-159 | Pulse change strobe |
| 6 | 160-191 | Always bright |
| 7 | 192-223 | Random strobe |
| 8 | 224-255 | Always bright |

Macro color

| serial number | Channel value | Basic features: |
| --- | --- | --- |
| 1 | 0 | empty |
| 2 | 1-179 | Macro color |
|  |  | Color rolling |
| 3 | 180-201 | Clockwise (from fast to slow) |
| 4 | 202-207 | Stop it |
| 5 | 208-229 | Counterclockwise (slow to fast) |
| 6 | 230-234 | Stop it |
|  |  | The colors are displayed randomly |
| 7 | 235-239 | fast |
| 8 | 240-244 | medium |
| 9 | 245-249 | slow |
| 10 | 250-255 | Stop it |

Background color

| serial number | Channel value | Basic features: |
| --- | --- | --- |
| 1 | 0-9 | Background color 0 (6 values are one color, the same below). |
| 2 | 10-15 | Background color 1 |
| 41 | 244-251 | Background color 40 |
| 42 | 252-255 | Background color 41 |

Control

| serial number | Channel value | Basic features: |
| --- | --- | --- |
| 1 | 0-99 | empty |
|  |  | Refresh rate |
| 2 | 100-105 | 1KHZ |
| 3 | 106-111 | 2KHZ |
| 4 | 112-117 | 4KHZ |
| 5 | 118-123 | 7KHZ |
| 6 | 124-129 | 9KHZ |
| 7 | 130-135 | 11KHZ |
| 8 | 136-141 | 14KHZ |
| 9 | 142-147 | 16KHZ |
| 10 | 148-153 | 18KHZ |
| 11 | 154-168 | 21KHZ |
| 12 | 169-180 | empty |
| 13 | 181-200 | empty |
|  |  | Dimming curve |
| 14 | 201-210 | Linear |
| 15 | 211-220 | square |
| 16 | 221-230 | Inverse square |
| 17 | 231-240 | S-shaped curve |
| 18 | 241-255 | empty |

Dim Control

| serial number | Channel value | Basic features: |
| --- | --- | --- |
| 1 | 0 | empty |
| 2 | 1-20 | standard |
| 3 | 21-40 | stage |
| 4 | 41-60 | TV |
| 5 | 61-80 | building |
| 6 | 81-100 | theater |
| 7 | 101-120 | Stage 2 |
| 8 | 121-255 | empty |

5. Frequently Asked Questions

For some common faults, corresponding solutions are proposed. Any unresolved issues should be handled by professionals. Disconnect the power supply before maintaining the fixture.

- The lamp beads don't turn on

- Check whether the voltage matching the lamp is installed;

- Check whether the power supply connection or control switch of the lamp is in poor contact;

- Check whether the power supply is insufficient;

- Check if the DMX512 controller has sent instructions.

2. The lamp does not accept the control of the console after normal reset

- Check whether the numeric start address value and function options of the luminaire are correct;

- Check whether the connection of the communication control line is correct, the communication line is too long or has been interrupted;

- Check whether the control equipment is failing, and check whether the signal amplifier connected in series is failing;

- Check whether the communication line is too long or if other devices interfere with each other;

- Optimize the wiring, shorten the length of the control signal line, and separate the high-voltage and low-voltage lines.

- Add a signal amplifier;

- The signal line adopts high-quality shielded twisted pair wire;

- Connect the signal termination resistor (120 ohms) at the end of the luminaire.

3. The light fixture cannot be started

- Check whether the power supply parameters are consistent with the lamps;

- Check that the lamps are in poor contact or fall off due to extrusion deformation, vibration of internal parts, moisture, etc. during long-distance transportation.

- Check whether the wires and connectors inside the lamp are falling off or loose.

- Check the electronic components of the lamp (such as electronic transformers, PCB boards, motor control boards, etc.) for looseness, short circuits, and burnouts.