| Product technical specification |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Product picture |  | Product dimension drawing |  |  |  |  |
| Lamp specification |  |  |  |  |  | parameter |
| Product name | 1400W BSWF IP 4in1 with CMY+CTO | Product model | GS-L1400QG |  | ·Power Supply:AC 100-240V,50/60Hz ·Power consumption:1800W ·Light Source:1400W ·LED life expectancy: 55000 Hours ·Color Temperature:2700K-6500K ·Diameter of lens:132mm ·Color Wheel:linear CMY color mixing,CTO color temperature adjustment (2700K-6500K), 1 color wheel:7 color+white light,Semi- color and rainbow effect; (adjustable rotating speed and direction) ·Gobo Wheel:1 static gobo wheel:7 gobos+open; 1 rotating gobo wheel:6 interchangeable gobos+open; gobo shake(adjustable rotating speed and direction) ·Animation wheel:Water ripple wheel ·Prism:6-facet prism,rotatable in both directions ·Framing shutters: 4x fast and smooth framing shutters, each individually controllable; each blade can block light completely; framing module can be rotated at ±60° ·Zoom: electronic linear zoom 2°-50° ·Focus:Variable motorized focus(0-100%) ·Frost:An independent frost effect ·Dimmer: 0-100% linear electronic dimmer ·Strobe: 1-25Hz/s,adjustable speed ·Movement:Pan: 540°,16 Bit movement resolution Tilt: 270°,16 Bit movement resolution Automatically accurate positioning ·Control mode:DMX512,RDM,Sound,Master/Slave,Auto ·DMX Channel:22/32/40 CH ·Protection Rating: IP65 ·Product Size:546*301*902mm,N.W.:66.6kg ·Carton size:720*470*1135mm,G.W:75.9kg |  |
|  |  | Lamp color | black |  |  |  |
| Net weight | 66.6kg | Gross weight | 75.9kg |  |  |  |
| Product size | 546*301*902mm | Package size | 720*470*1135mm |  |  |  |
| Degree of protection | IP65 | Lamp material | Aluminum alloy, high temperature and ultraviolet resistant engineering plastics |  |  |  |
| Optical system |  | Effect system |  |  |  |  |
| Light source information | 1400W White LED | Color wheel | linear CMY color mixing,CTO color temperature adjustment (2700K-6500K), 1 color wheel:7 color+white light,Semi- color and rainbow effect, (adjustable rotating speed and direction) |  |  |  |
| Light source life | 55000 Hours | Gobo wheel | 1 static gobo wheel:7 gobos+open; 1 rotating gobo wheel:6 interchangeable gobos+open; gobo shake(adjustable rotating speed and direction) |  |  |  |
| Color temperature | 2700K-6500K | Animation wheel | Water ripple wheel |  |  |  |
| Zoom | electronic linear zoom 2°-50° | Prism | 6-facet prism,rotatable in both directions |  |  |  |
| Diameter of lens | 132mm | Framing system | 4x fast and smooth framing shutters, each individually controllable; each blade can block light completely; framing module can be rotated at ±60° |  |  |  |
| Focus | Variable motorized focus(0-100%) | Strobe | 1-25Hz/s,adjustable speed |  |  |  |
| Frost | An independent frost effect | Dimmer | 0-100% linear electronic dimmer |  |  |  |
| Electrical connection |  | Control system |  |  |  |  |
| Input voltage | AC 100V-240V, 50/60Hz | Movement | Pan: 540°.16 Bit movement resolution; Tilt: 270°,16 Bit movement resolution; Automatically accurate positioning |  |  |  |
| Whole lamp power | 1800W | Communication design | DMX wired / wireless transmission, convenient and fast software upgrade through DMX data cable. |  |  |  |
| Signal connection | 3/5 pin IP65 XLR I / O | Control Mode | DMX512,RDM,Sound,Auto,Manual,Master/Slave |  |  |  |
| Power connection | IP66 power head input | DMX Channel | 22/32/40 CH |  |  |  |
| Display | LCD digital display,support Chinese & English,Lamps and lanterns can be queried, and the service time of bulbs is convenient for maintenance |  |  |  |  |  |
| Other |  |  |  |  |  |  |
| Gobo+ Color |  |  |  | Effect |  |  |
| ROTATING GOBO WHEEL : STATIC GOBO WHEEL : COLOR WHEEL : |  |  |  |  |  |  |