EPIC STROBE

LED Combinable STROBE light

This product manual contains important information about the safe installation and use of this projector. Please read and follow these instructions carefully and keep this manual in a safe place for future reference.

User manual

Please read the instructions carefully before use

TABLE OF CONTENTS

- Dimension	3

- Safety Information	3

- Preparing for installation	5

- Parts identification and terminology	6

- Installation	7

- Connecting AC Power	8

- Configuring the fixture	9

- Connecting to a DMX control device	10

- Configuring the fixture for DMX control	11

- Cleaning	12

- DMX Protocols	12

- Control Menu	99

- Specification	101

STATEMENT

The product has well capability and intact packing when leave factory. All of the user should comply with warning item and manual, any misuse cause of the damages are not included in our guarantee, and also can not be responsible for any malfunction & problem owing to ignore the manual.

1. Dimension

2. Safety Information

WARNING! Read the safety precautions in this section before unpacking, installing, powering or operating this product.

This luminaries are multi-environmental fixtures with an IP-rating of 65, intended for professional use only. They are not suitable for household use.

Review the following safety precautions carefully before installing or operating the fixture. This fixture must be installed in accordance with the applicable installation code by a person familiar with the construction and operation of the fixture and the hazards involved.

Preventing electric shock

WARNING! Risk of electric shock.

- Always power off/unplug the fixture before removing any covers.

- Ensure that the power is turned off when connecting the fixture to the AC mains supply.

- Ensure that the fixture is electrically connected to earth (ground).

- Do not apply power if the fixture is in any way damaged.

- Do not immerse the fixture in water or liquid.

Preventing burns and fire

WARNING! Take measures to prevent burns and fire.

- Install in a location that prevents accidental contact with the fixture.

- Install only in a well-ventilated space.

- Install at least 0.3 m (12 in.) away from objects to be illuminated.

- Install only in accordance with applicable building codes.

- Ensure a minimum clearance of 0.1 m (4 in.) around the cooling fans.

- Do not paint, cover or modify the fixture.

- Keep all flammable materials away from the fixture.

- Allow the fixture to cool for 15 minutes after operation, before touching it.

- CAUTION: Exterior surface temperature after 5 min. operation = 45 °C (113 °F). Steady state = 60 °C (140 °F).

Avoid personal injury

WARNING! Take measure to prevent personal injury.

- Do not look directly at the light source from close range.

- Take precautions to prevent injury due to falls when working at height.

- For permanent installation, ensure that the fixture is securely fastened to a load-bearing surface with suitable corrosion-resistant hardware.

- For temporary installation with clamps, ensure that the quarter-turn fasteners are turned fully and secured with a suitable safety cable. The cable must be approved for a safe working load (SWL) of 10 times the weight of the fixture, and it must have a minimum gauge of 3 mm.

3. Preparing for installation

Unpack the fixture and inspect it to ensure that it has not been damaged during transport.

The fixture is shipped with two quarter-turn brackets, that can be used to mount the fixture at elevation.

The fixture is IP65-rated, and is designed for use in wet locations. This means that it is protected from:

- Dust, to the degree that dust cannot enter the fixture in sufficient quantities as to interfere with its operation.

- Lower pressure jets of water from any direction.

When selecting a location for the fixture, ensure that:

- It is situated away from public thoroughfares and protected from contact with people.

- It is not immersed in water or exposed to high-pressure water jets.

- It has adequate ventilation.

4. Parts identification and terminology

LED white strobe tubes

LED RGB 3in1 panels

Adjustable Handle

Waterproof fans

SEETRONIC True1 Power In&Out

SEETRONIC DMX In&Out

SEETRONIC ArtNet RJ45 In&Out

LCD Touch Screen

Combine Construct

5. Installation

The fixture may be installed in any orientation, but if installed horizontally with a downward beam-angle, water can potentially pool in the fan wells. Under normal operation the moisture will evaporate. However, in locations with high rainfall, you may wish to fabricate a rain shield above the fixture, or modify the position and orientation of the fixture to minimize pooling.

Two quarter-turn brackets are supplied with the fixture if it is to be flown above the ground. Rig the fixture to a support truss or structure using the supplied brackets and suitable clamps.

Fasten a safety cable (not shown) between the support structure and the attachment point on the fixture. The safety cable must be able to bear at least 10 times the weight of the fixture.

Please note that this is only part of the combination and hanging method, when using one hook as a hanging point, a maximum of 4 lights can be connected.

6. Connecting AC Power

The fixture can operate on any 100–240 V,

50/60 Hz AC mains power supply. It draws approximately 2 amps at full power. For permanent installation, have a qualified electrician wire the mains cable directly to a suitable branch circuit.

The junction’s ingress protection (IP) rating must be suitable for the location. For temporary installation, the mains cable may be fitted with a grounded connector intended for exterior use.

When installing standard type C circuit breakers there will be no limitations due to the fixture in-rush current. Due to the nominal current of the fixture, ensure that no more than:

4 fixtures are connected through the same type C, 10A circuit breaker. 7 fixtures are connected through the same type C, 16A circuit breaker.

The fixture must be grounded/earthed and be able to be isolated from AC power. The AC power supply must incorporate a fuse or circuit breaker for fault protection.

After connecting the fixture to power, run the on-board test, using the “Fixture Text” menu, to ensure that the fixture and each LED are functioning correctly. See “Control menu” on page 63.

CAUTION: Do not open the fixture to replace the supplied power cable, or connect the fixture to an electrical dimmer system, as this can damage it.

7. Configuring the fixture

Set up the fixture using the control panel and LCD display at the arm side of the fixture.

Navigate the menus and options using the arrow buttons and select items using the Enter button. The options available are listed in “Control menu” on page 63. After powering on, the display shows the currently selected operating mode and other information.

The fixture is set by default to be controlled in DMX mode.

Master/Slave configuration

You can set a fixture to operate as master fixture to another fixture (which then becomes a slave fixture), or an entire group of fixtures (which then becomes slave fixtures). The assigned slave fixture(s) will mimic the settings of the master fixture. Use the “Auto Program -> Auto Color / Auto Fade” menu to set your fixture as master fixture, then other fixtures set to DMX mode as slave fixture.

Setting a static color manually

The fixture can be configured to display a predefined and static color using the “Manual Color” (see “Control menu” on page 63).

It may suit your needs when you without a DMX controller to do the color

mixing.

Using stand-alone operation

Stand-alone operation is where the fixture is not connected to a control device, but is preprogrammed with 2 modes (Auto Color, Auto Fade), that play continuously in a loop, the run speed of “Auto Color”, “Auto Fade” are adjustable.

To define a stand-alone program, use the “Auto Program” menus (see “Control menu” on page 63).

8. Connecting to a DMX control device

The fixture is controllable using a DMX control device and it can be connected using a DMX cable.

If using a cabled DMX system, connect the DMX in cable (with male 3-pin XLR plug) and out cable (with female 3-pin XLR plug) to the DMX data link.

Terminate the DMX out cable of the last fixture in the data link. For outdoor installations, use only IP-rated XLR connectors suitable for outdoor use.

The DMX512 is widely used in intelligent lighting control, with a DMX 512 controller. connect several lights together, DMX in and DMX out, 3 pin XLR connectors: Pin 1: GND, Pin 2: Negative signal (-), Pin 3: Positive signal (+)

9. Configuring the fixture for DMX control

About DMX

The fixture can be controlled using signals sent by a DMX controller on a number of channels (which varies depending on the DMX mode that has been set).

The first channel used to receive data from a DMX control device is known as the DMX start address. Each fixture must have a DMX start address set. For example, if a fixture has a DMX address of 10 and it is in 4-channel DMX mode, then it uses channels 10, 11,12 and 13. The following fixture in the DMX chain could then be set to a DMX address of 14. If two or more DMX fixtures of the same type have the same DMX address, then they will mimic each other’s behaviour. Incorrect settings will result in unpredictable responses to the lighting controller.

Setting the DMX address

The DMX address can be seen on the main screen. To change the address setting, press the up arrow to increase the address, or the down arrow to decrease the setting. When the desired address is displayed, press Enter to save the setting.

Note that channel spacing is determined by the DMX mode.

See the “DMX protocols” on page 12 for specific DMX control values.

Setting the DMX mode

Using the “DMX Channel Mode” menu available from the control panel, specify the DMX mode that provides the fixture controls that you require, confirm chosen mode by pressing ‘Enter’.

10. Cleaning

To maintain optimal performance, regular cleaning is essential. Cleaning schedules will vary depending on the operating environment, and the installation should therefore be checked at frequent intervals within the first few weeks of operation to see whether cleaning is necessary. This procedure will allow you to assess cleaning requirements in your par- ticular situation. Clean the fixture using a soft cloth dampened with a solution of water and a mild detergent. Do not use products that contain solvents, abrasives or caustic agents for cleaning, as they can cause damage to both hardware, cables and connectors.

11. DMX protocols

| 15CH SIMPLE MODE |  |  |  |
| --- | --- | --- | --- |
| CH | Function | Value | Remark |
| 1 | Master Dimmer | 000-255 | 0-100% |
| 2 | Centric Tube Dimmer | 000-255 | 0-100% |
| 3 | Centric Tube Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 4 | Centric Tube Duration | 000-255 | Only affects to normal strobe (025-064) |
| 5 | Single Tube (1-4) Dimmer | 000-255 | 0-100% |
| 6 | Single Tube (1-4) Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 7 | Single Tube (1-4) Duration | 000-255 | Only affects to normal strobe (025-064) |
| 8 | RGB Panels (1+2) Dimmer | 000-255 | 0-100% |
| 9 | RGB Panels (1+2) Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 10 | RGB Panels (1+2) Duration | 000-255 | Only affects to normal strobe (025-064) |
| 11 | RGB Panels (1+2) Red | 000-255 | 0-100% |
| 12 | RGB Panels (1+2) Green | 000-255 | 0-100% |
| 13 | RGB Panels (1+2) Blue | 000-255 | 0-100% |
| 14 | RGB Panels (1+2) CTC | 000-255 | CTC |
| 15 | Device Settings (Hold 3s to activate) | 000-019 | No function |
|  |  | 020-024 | Display Backlight On |
|  |  | 025-029 | Display Backlight Off |
|  |  | 105-109 | Fan Auto |
|  |  | 110-114 | Fan Silent |
|  |  | 115-119 | Fan Off |
|  |  | 120-124 | Fan High Power |
|  |  | 130-134 | Invert RGB Panel 1 Off |
|  |  | 135-139 | Invert RGB Panel 1 On |
|  |  | 140-144 | Invert RGB Panel 2 Off |
|  |  | 145-149 | Invert RGB Panel 2 On |
|  |  | 150-154 | Invert Centric Tube Off |
|  |  | 155-159 | Invert Centric Tube On |
|  |  | 160-164 | Invert Centric Tube 1 Off |
|  |  | 165-169 | Invert Centric Tube 1 On |
|  |  | 170-174 | Invert Centric Tube 2 Off |
|  |  | 175-179 | Invert Centric Tube 2 On |
|  |  | 180-184 | Invert Centric Tube 3 Off |
|  |  | 185-189 | Invert Centric Tube 3 On |
|  |  | 190-194 | Invert Centric Tube 4 Off |
|  |  | 195-199 | Invert Centric Tube 4 On |
|  |  | 200-204 | Swap RGB Panels Off |
|  |  | 205-209 | Swap RGB Panels On |
|  |  | 240-244 | Factory Reset |

| 16CH SIMPLE MODE |  |  |  |
| --- | --- | --- | --- |
| CH | Function | Value | Remark |
| 1 | Master Dimmer | 000-255 | 0-100% |
| 2 | Centric Tube Dimmer | 000-255 | 0-100% |
| 3 | Centric Tube Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 4 | Centric Tube Duration | 000-255 | Only affects to normal strobe (025-064) |
| 5 | Single Tube (1-4) Dimmer | 000-255 | 0-100% |
| 6 | Single Tube (1-4) Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 7 | Single Tube (1-4) Duration | 000-255 | Only affects to normal strobe (025-064) |
| 8 | RGB Panels (1+2) Dimmer | 000-255 | 0-100% |
| 9 | RGB Panels (1+2) Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 10 | RGB Panels (1+2) Duration | 000-255 | Only affects to normal strobe (025-064) |
| 11 | RGB Panels (1+2) Red | 000-255 | 0-100% |
| 12 | RGB Panels (1+2) Green | 000-255 | 0-100% |
| 13 | RGB Panels (1+2) Blue | 000-255 | 0-100% |
| 14 | RGB Panels (1+2) White | 000-255 | 0-100% |
| 15 | RGB Panels (1+2) CTC | 000-255 | CTC |
| 16 | Device Settings (Hold 3s to activate) | 000-019 | No function |
|  |  | 020-024 | Display Backlight On |
|  |  | 025-029 | Display Backlight Off |
|  |  | 105-109 | Fan Auto |
|  |  | 110-114 | Fan Silent |
|  |  | 115-119 | Fan Off |
|  |  | 120-124 | Fan High Power |
|  |  | 130-134 | Invert RGB Panel 1 Off |
|  |  | 135-139 | Invert RGB Panel 1 On |
|  |  | 140-144 | Invert RGB Panel 2 Off |
|  |  | 145-149 | Invert RGB Panel 2 On |
|  |  | 150-154 | Invert Centric Tube Off |
|  |  | 155-159 | Invert Centric Tube On |
|  |  | 160-164 | Invert Centric Tube 1 Off |
|  |  | 165-169 | Invert Centric Tube 1 On |
|  |  | 170-174 | Invert Centric Tube 2 Off |
|  |  | 175-179 | Invert Centric Tube 2 On |
|  |  | 180-184 | Invert Centric Tube 3 Off |
|  |  | 185-189 | Invert Centric Tube 3 On |
|  |  | 190-194 | Invert Centric Tube 4 Off |
|  |  | 195-199 | Invert Centric Tube 4 On |
|  |  | 200-204 | Swap RGB Panels Off |
|  |  | 205-209 | Swap RGB Panels On |
|  |  | 240-244 | Factory Reset |

| 39CH EASY MODE |  |  |  |
| --- | --- | --- | --- |
| CH | Function | Value | Remark |
| 1 | Master Dimmer | 000-255 | 0-100% |
| 2 | Centric Tube Dimmer | 000-255 | 0-100% |
| 3 | Centric Tube Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 4 | Centric Tube Duration | 000-255 | Only affects to normal strobe (025-064) |
| 5 | Centric Tube Pattern Dimmer | 000-255 | 0-100% |
| 6 | Centric Tube Pattern Selection | 000-001 | No function |
|  |  | 002-143 | One pattern per value |
|  |  | 144-255 | No function |
| 7 | Centric Tube Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 8 | Centric Tube Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 9 | Centric Tube Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 10 | Reserve | 000-255 | Reserve |
| 11 | Single Tube (1-4) Dimmer | 000-255 | 0-100% |
| 12 | Single Tube (1-4) Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 13 | Single Tube (1-4) Duration | 000-255 | Only affects to normal strobe (025-064) |
| 14 | Single Tube (1-4) Pattern Dimmer | 000-255 | 0-100% |
| 15 | Single Tube (1-4) Pattern Selection | 000-001 | No function |
|  |  | 002-143 | One pattern per value |
|  |  | 144-255 | No function |
| 16 | Reserve | 000-255 | Reserve |
| 17 | Reserve | 000-255 | Reserve |
| 18 | Single Tube (1-4) Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 19 | Single Tube (1-4) Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 20 | Single Tube (1-4) Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 21 | Reserve | 000-255 | Reserve |
| 22 | RGB Panels (1+2) Dimmer | 000-255 | 0-100% |
| 23 | RGB Panels (1+2) Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 24 | RGB Panels (1+2) Duration | 000-255 | Only affects to normal strobe (025-064) |
| 25 | RGB Panels (1+2) Red | 000-255 | 0-100% |
| 26 | RGB Panels (1+2) Green | 000-255 | 0-100% |
| 27 | RGB Panels (1+2) Blue | 000-255 | 0-100% |
| 28 | RGB Panels (1+2) CTC | 000-255 | CTC |
| 29 | RGB Panels (1+2) Pattern Dimmer | 000-255 | 0-100% |
| 30 | RGB Panels (1+2) Pattern Selection | 000-001 | No function |
|  |  | 002-079 | One pattern per value |
|  |  | 080-255 | No function |
| 31 | RGB Panels (1+2) Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 32 | RGB Panels (1+2) Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 33 | RGB Panels (1+2) Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 34 | Reserve | 000-255 | Reserve |
| 35 | RGB Panels (1+2) Pattern Red | 000-255 | 0-100% |
| 36 | RGB Panels (1+2) Pattern Green | 000-255 | 0-100% |
| 37 | RGB Panels (1+2) Pattern Blue | 000-255 | 0-100% |
| 38 | RGB Panels (1+2) Pattern CTC | 000-255 | Pattern CTC |
| 39 | Device Settings (Hold 3s to activate) | 000-019 | No function |
|  |  | 020-024 | Display Backlight On |
|  |  | 025-029 | Display Backlight Off |
|  |  | 105-109 | Fan Auto |
|  |  | 110-114 | Fan Silent |
|  |  | 115-119 | Fan Off |
|  |  | 120-124 | Fan High Power |
|  |  | 130-134 | Invert RGB Panel 1 Off |
|  |  | 135-139 | Invert RGB Panel 1 On |
|  |  | 140-144 | Invert RGB Panel 2 Off |
|  |  | 145-149 | Invert RGB Panel 2 On |
|  |  | 150-154 | Invert Centric Tube Off |
|  |  | 155-159 | Invert Centric Tube On |
|  |  | 160-164 | Invert Centric Tube 1 Off |
|  |  | 165-169 | Invert Centric Tube 1 On |
|  |  | 170-174 | Invert Centric Tube 2 Off |
|  |  | 175-179 | Invert Centric Tube 2 On |
|  |  | 180-184 | Invert Centric Tube 3 Off |
|  |  | 185-189 | Invert Centric Tube 3 On |
|  |  | 190-194 | Invert Centric Tube 4 Off |
|  |  | 195-199 | Invert Centric Tube 4 On |
|  |  | 200-204 | Swap RGB Panels Off |
|  |  | 205-209 | Swap RGB Panels On |
|  |  | 240-244 | Factory Reset |

| 71CH STANDARD MODE |  |  |  |
| --- | --- | --- | --- |
| CH | Function | Value | Remark |
| 1 | Master Dimmer | 000-255 | 0-100% |
| 2 | Centric Tube Dimmer | 000-255 | 0-100% |
| 3 | Centric Tube Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 4 | Centric Tube Duration | 000-255 | Only affects to normal strobe (025-064) |
| 5 | Centric Tube Pattern Dimmer | 000-255 | 0-100% |
| 6 | Centric Tube Pattern Selection | 000-001 | No function |
|  |  | 002-143 | One pattern per value |
|  |  | 144-255 | No function |
| 7 | Centric Tube Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 8 | Centric Tube Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 9 | Centric Tube Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 10 | Reserve | 000-255 | Reserve |
| 11 | Single Tube 1 Dimmer | 000-255 | 0-100% |
| 12 | Single Tube 1 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 13 | Single Tube 1 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 14 | Single Tube 1 Pattern Dimmer | 000-255 | 0-100% |
| 15 | Single Tube 1 Pattern Selection | 000-001 | No function |
|  |  | 002-143 | One pattern per value |
|  |  | 144-255 | No function |
| 16 | Single Tube 1 Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 17 | Single Tube 1 Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 18 | Single Tube 1 Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 19 | Reserve | 000-255 | Reserve |
| 20 | Single Tube 2 Dimmer | 000-255 | 0-100% |
| 21 | Single Tube 2 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 22 | Single Tube 2 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 23 | Single Tube 2 Pattern Dimmer | 000-255 | 0-100% |
| 24 | Single Tube 2 Pattern Selection | 000-001 | No function |
|  |  | 002-143 | One pattern per value |
|  |  | 144-255 | No function |
| 25 | Single Tube 2 Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 26 | Single Tube 2 Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 27 | Single Tube 2 Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 28 | Reserve | 000-255 | Reserve |
| 29 | Single Tube 3 Dimmer | 000-255 | 0-100% |
| 30 | Single Tube 3 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 31 | Single Tube 3 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 32 | Single Tube 3 Pattern Dimmer | 000-255 | 0-100% |
| 33 | Single Tube 3 Pattern Selection | 000-001 | No function |
|  |  | 002-143 | One pattern per value |
|  |  | 144-255 | No function |
| 34 | Single Tube 3 Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 35 | Single Tube 3 Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 36 | Single Tube 3 Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 37 | Reserve | 000-255 | Reserve |
| 38 | Single Tube 4 Dimmer | 000-255 | 0-100% |
| 39 | Single Tube 4 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 40 | Single Tube 4 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 41 | Single Tube 4 Pattern Dimmer | 000-255 | 0-100% |
| 42 | Single Tube 4 Pattern Selection | 000-001 | No function |
|  |  | 002-143 | One pattern per value |
|  |  | 144-255 | No function |
| 43 | Single Tube 4 Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 44 | Single Tube 4 Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 45 | Single Tube 4 Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 46 | Reserve | 000-255 | Reserve |
| 47 | Single Tube (1-4) Dimmer | 000-255 | 0-100% |
| 48 | Single Tube (1-4) Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 49 | Single Tube (1-4) Duration | 000-255 | Only affects to normal strobe (025-064) |
| 50 | Single Tube (1-4) Pattern Dimmer | 000-255 | 0-100% |
| 51 | Single Tube (1-4) Pattern Selection | 000-001 | No function |
|  |  | 002-143 | One pattern per value |
|  |  | 144-255 | No function |
| 52 | Reserve | 000-255 | Reserve |
| 53 | RGB Panels (1+2) Dimmer | 000-255 | 0-100% |
| 54 | RGB Panels (1+2) Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 55 | RGB Panels (1+2) Duration | 000-255 | Only affects to normal strobe (025-064) |
| 56 | RGB Panels (1+2) Red | 000-255 | 0-100% |
| 57 | RGB Panels (1+2) Green | 000-255 | 0-100% |
| 58 | RGB Panels (1+2) Blue | 000-255 | 0-100% |
| 59 | RGB Panels (1+2) CTC | 000-255 | CTC |
| 60 | RGB Panels (1+2) Backgroud Color | 000-255 | Macro background color |
| 61 | RGB Panels (1+2) Backgroud Color Fade | 000-255 | Macro background color fade |
| 62 | RGB Panels (1+2) Pattern Dimmer | 000-255 | 0-100% |
| 63 | RGB Panels (1+2) Pattern Selection | 000-001 | No function |
|  |  | 002-079 | One pattern per value |
|  |  | 080-255 | No function |
| 64 | RGB Panels (1+2) Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 65 | RGB Panels (1+2) Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 66 | RGB Panels (1+2) Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 67 | Reserve | 000-255 | Reserve |
| 68 | RGB Panels (1+2) Pattern Red | 000-255 | 0-100% |
| 69 | RGB Panels (1+2) Pattern Green | 000-255 | 0-100% |
| 70 | RGB Panels (1+2) Pattern Blue | 000-255 | 0-100% |
| 71 | Device Settings (Hold 3s to activate) | 000-019 | No function |
|  |  | 020-024 | Display Backlight On |
|  |  | 025-029 | Display Backlight Off |
|  |  | 105-109 | Fan Auto |
|  |  | 110-114 | Fan Silent |
|  |  | 115-119 | Fan Off |
|  |  | 120-124 | Fan High Power |
|  |  | 130-134 | Invert RGB Panel 1 Off |
|  |  | 135-139 | Invert RGB Panel 1 On |
|  |  | 140-144 | Invert RGB Panel 2 Off |
|  |  | 145-149 | Invert RGB Panel 2 On |
|  |  | 150-154 | Invert Centric Tube Off |
|  |  | 155-159 | Invert Centric Tube On |
|  |  | 160-164 | Invert Centric Tube 1 Off |
|  |  | 165-169 | Invert Centric Tube 1 On |
|  |  | 170-174 | Invert Centric Tube 2 Off |
|  |  | 175-179 | Invert Centric Tube 2 On |
|  |  | 180-184 | Invert Centric Tube 3 Off |
|  |  | 185-189 | Invert Centric Tube 3 On |
|  |  | 190-194 | Invert Centric Tube 4 Off |
|  |  | 195-199 | Invert Centric Tube 4 On |
|  |  | 200-204 | Swap RGB Panels Off |
|  |  | 205-209 | Swap RGB Panels On |
|  |  | 240-244 | Factory Reset |

| 112CH PURE MODE |  |  |  |
| --- | --- | --- | --- |
| CH | Function | Value | Remark |
| 1 | Master Dimmer | 000-255 | 0-100% |
| 2 | Centric Tube Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 3 | Centric Tube Duration | 000-255 | Only affects to normal strobe (025-064) |
| 4 | Centric Tube Pixel 1 Intensity | 000-255 | 0-100% |
| 5 | Centric Tube Pixel 2 Intensity | 000-255 | 0-100% |
| 6 | Centric Tube Pixel 3 Intensity | 000-255 | 0-100% |
| 7 | Centric Tube Pixel 4 Intensity | 000-255 | 0-100% |
| 8 | Centric Tube Pixel 5 Intensity | 000-255 | 0-100% |
| 9 | Centric Tube Pixel 6 Intensity | 000-255 | 0-100% |
| 10 | Centric Tube Pixel 7 Intensity | 000-255 | 0-100% |
| 11 | Centric Tube Pixel 8 Intensity | 000-255 | 0-100% |
| 12 | Centric Tube Pixel 9 Intensity | 000-255 | 0-100% |
| 13 | Centric Tube Pixel 10 Intensity | 000-255 | 0-100% |
| 14 | Centric Tube Pixel 11 Intensity | 000-255 | 0-100% |
| 15 | Centric Tube Pixel 12 Intensity | 000-255 | 0-100% |
| 16 | Single Tube 1 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 17 | Single Tube 1 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 18 | Single Tube 1 Pixel 1 Intensity | 000-255 | 0-100% |
| 19 | Single Tube 1 Pixel 2 Intensity | 000-255 | 0-100% |
| 20 | Single Tube 1 Pixel 3 Intensity | 000-255 | 0-100% |
| 21 | Single Tube 1 Pixel 4 Intensity | 000-255 | 0-100% |
| 22 | Single Tube 1 Pixel 5 Intensity | 000-255 | 0-100% |
| 23 | Single Tube 1 Pixel 6 Intensity | 000-255 | 0-100% |
| 24 | Single Tube 1 Pixel 7 Intensity | 000-255 | 0-100% |
| 25 | Single Tube 1 Pixel 8 Intensity | 000-255 | 0-100% |
| 26 | Single Tube 1 Pixel 9 Intensity | 000-255 | 0-100% |
| 27 | Single Tube 1 Pixel 10 Intensity | 000-255 | 0-100% |
| 28 | Single Tube 1 Pixel 11 Intensity | 000-255 | 0-100% |
| 29 | Single Tube 1 Pixel 12 Intensity | 000-255 | 0-100% |
| 30 | Single Tube 2 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 31 | Single Tube 2 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 32 | Single Tube 2 Pixel 1 Intensity | 000-255 | 0-100% |
| 33 | Single Tube 2 Pixel 2 Intensity | 000-255 | 0-100% |
| 34 | Single Tube 2 Pixel 3 Intensity | 000-255 | 0-100% |
| 35 | Single Tube 2 Pixel 4 Intensity | 000-255 | 0-100% |
| 36 | Single Tube 2 Pixel 5 Intensity | 000-255 | 0-100% |
| 37 | Single Tube 2 Pixel 6 Intensity | 000-255 | 0-100% |
| 38 | Single Tube 2 Pixel 7 Intensity | 000-255 | 0-100% |
| 39 | Single Tube 2 Pixel 8 Intensity | 000-255 | 0-100% |
| 40 | Single Tube 2 Pixel 9 Intensity | 000-255 | 0-100% |
| 41 | Single Tube 2 Pixel 10 Intensity | 000-255 | 0-100% |
| 42 | Single Tube 2 Pixel 11 Intensity | 000-255 | 0-100% |
| 43 | Single Tube 2 Pixel 12 Intensity | 000-255 | 0-100% |
| 44 | Single Tube 3 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 45 | Single Tube 3 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 46 | Single Tube 3 Pixel 1 Intensity | 000-255 | 0-100% |
| 47 | Single Tube 3 Pixel 2 Intensity | 000-255 | 0-100% |
| 48 | Single Tube 3 Pixel 3 Intensity | 000-255 | 0-100% |
| 49 | Single Tube 3 Pixel 4 Intensity | 000-255 | 0-100% |
| 50 | Single Tube 3 Pixel 5 Intensity | 000-255 | 0-100% |
| 51 | Single Tube 3 Pixel 6 Intensity | 000-255 | 0-100% |
| 52 | Single Tube 3 Pixel 7 Intensity | 000-255 | 0-100% |
| 53 | Single Tube 3 Pixel 8 Intensity | 000-255 | 0-100% |
| 54 | Single Tube 3 Pixel 9 Intensity | 000-255 | 0-100% |
| 55 | Single Tube 3 Pixel 10 Intensity | 000-255 | 0-100% |
| 56 | Single Tube 3 Pixel 11 Intensity | 000-255 | 0-100% |
| 57 | Single Tube 3 Pixel 12 Intensity | 000-255 | 0-100% |
| 58 | Single Tube 4 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 59 | Single Tube 4 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 60 | Single Tube 4 Pixel 1 Intensity | 000-255 | 0-100% |
| 61 | Single Tube 4 Pixel 2 Intensity | 000-255 | 0-100% |
| 62 | Single Tube 4 Pixel 3 Intensity | 000-255 | 0-100% |
| 63 | Single Tube 4 Pixel 4 Intensity | 000-255 | 0-100% |
| 64 | Single Tube 4 Pixel 5 Intensity | 000-255 | 0-100% |
| 65 | Single Tube 4 Pixel 6 Intensity | 000-255 | 0-100% |
| 66 | Single Tube 4 Pixel 7 Intensity | 000-255 | 0-100% |
| 67 | Single Tube 4 Pixel 8 Intensity | 000-255 | 0-100% |
| 68 | Single Tube 4 Pixel 9 Intensity | 000-255 | 0-100% |
| 69 | Single Tube 4 Pixel 10 Intensity | 000-255 | 0-100% |
| 70 | Single Tube 4 Pixel 11 Intensity | 000-255 | 0-100% |
| 71 | Single Tube 4 Pixel 12 Intensity | 000-255 | 0-100% |
| 72 | RGB Panel 1 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 73 | RGB Panel 1 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 74 | RGB Panel 1 Red 1 | 000-255 | 0-100% |
| 75 | RGB Panel 1 Green 1 | 000-255 | 0-100% |
| 76 | RGB Panel 1 Blue 1 | 000-255 | 0-100% |
| 77 | RGB Panel 1 Red 2 | 000-255 | 0-100% |
| 78 | RGB Panel 1 Green 2 | 000-255 | 0-100% |
| 79 | RGB Panel 1 Blue 2 | 000-255 | 0-100% |
| 80 | RGB Panel 1 Red 3 | 000-255 | 0-100% |
| 81 | RGB Panel 1 Green 3 | 000-255 | 0-100% |
| 82 | RGB Panel 1 Blue 3 | 000-255 | 0-100% |
| 83 | RGB Panel 1 Red 4 | 000-255 | 0-100% |
| 84 | RGB Panel 1 Green 4 | 000-255 | 0-100% |
| 85 | RGB Panel 1 Blue 4 | 000-255 | 0-100% |
| 86 | RGB Panel 1 Red 5 | 000-255 | 0-100% |
| 87 | RGB Panel 1 Green 5 | 000-255 | 0-100% |
| 88 | RGB Panel 1 Blue 5 | 000-255 | 0-100% |
| 89 | RGB Panel 1 Red 6 | 000-255 | 0-100% |
| 90 | RGB Panel 1 Green 6 | 000-255 | 0-100% |
| 91 | RGB Panel 1 Blue 6 | 000-255 | 0-100% |
| 92 | RGB Panel 2 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 93 | RGB Panel 2 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 94 | RGB Panel 2 Red 1 | 000-255 | 0-100% |
| 95 | RGB Panel 2 Green 1 | 000-255 | 0-100% |
| 96 | RGB Panel 2 Blue 1 | 000-255 | 0-100% |
| 97 | RGB Panel 2 Red 2 | 000-255 | 0-100% |
| 98 | RGB Panel 2 Green 2 | 000-255 | 0-100% |
| 99 | RGB Panel 2 Blue 2 | 000-255 | 0-100% |
| 100 | RGB Panel 2 Red 3 | 000-255 | 0-100% |
| 101 | RGB Panel 2 Green 3 | 000-255 | 0-100% |
| 102 | RGB Panel 2 Blue 3 | 000-255 | 0-100% |
| 103 | RGB Panel 2 Red 4 | 000-255 | 0-100% |
| 104 | RGB Panel 2 Green 4 | 000-255 | 0-100% |
| 105 | RGB Panel 2 Blue 4 | 000-255 | 0-100% |
| 106 | RGB Panel 2 Red 5 | 000-255 | 0-100% |
| 107 | RGB Panel 2 Green 5 | 000-255 | 0-100% |
| 108 | RGB Panel 2 Blue 5 | 000-255 | 0-100% |
| 109 | RGB Panel 2 Red 6 | 000-255 | 0-100% |
| 110 | RGB Panel 2 Green 6 | 000-255 | 0-100% |
| 111 | RGB Panel 2 Blue 6 | 000-255 | 0-100% |
| 112 | Device Settings (Hold 3s to activate) | 000-019 | No function |
|  |  | 020-024 | Display Backlight On |
|  |  | 025-029 | Display Backlight Off |
|  |  | 105-109 | Fan Auto |
|  |  | 110-114 | Fan Silent |
|  |  | 115-119 | Fan Off |
|  |  | 120-124 | Fan High Power |
|  |  | 130-134 | Invert RGB Panel 1 Off |
|  |  | 135-139 | Invert RGB Panel 1 On |
|  |  | 140-144 | Invert RGB Panel 2 Off |
|  |  | 145-149 | Invert RGB Panel 2 On |
|  |  | 150-154 | Invert Centric Tube Off |
|  |  | 155-159 | Invert Centric Tube On |
|  |  | 160-164 | Invert Centric Tube 1 Off |
|  |  | 165-169 | Invert Centric Tube 1 On |
|  |  | 170-174 | Invert Centric Tube 2 Off |
|  |  | 175-179 | Invert Centric Tube 2 On |
|  |  | 180-184 | Invert Centric Tube 3 Off |
|  |  | 185-189 | Invert Centric Tube 3 On |
|  |  | 190-194 | Invert Centric Tube 4 Off |
|  |  | 195-199 | Invert Centric Tube 4 On |
|  |  | 200-204 | Swap RGB Panels Off |
|  |  | 205-209 | Swap RGB Panels On |
|  |  | 240-244 | Factory Reset |

| 209CH PURE MODE |  |  |  |
| --- | --- | --- | --- |
| CH | Function | Value | Remark |
| 1 | Master Dimmer | 000-255 | 0-100% |
| 2 | Master Dimmer fine | 000-255 | 0-100% |
| 3 | Centric Tube Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 4 | Centric Tube Duration | 000-255 | Only affects to normal strobe (025-064) |
| 5 | Centric Tube Pixel 1 Intensity | 000-255 | 0-100% |
| 6 | Centric Tube Pixel 1 Intensity fine | 000-255 | 0-100% |
| 7 | Centric Tube Pixel 2 Intensity | 000-255 | 0-100% |
| 8 | Centric Tube Pixel 2 Intensity fine | 000-255 | 0-100% |
| 9 | Centric Tube Pixel 3 Intensity | 000-255 | 0-100% |
| 10 | Centric Tube Pixel 3 Intensity fine | 000-255 | 0-100% |
| 11 | Centric Tube Pixel 4 Intensity | 000-255 | 0-100% |
| 12 | Centric Tube Pixel 4 Intensity fine | 000-255 | 0-100% |
| 13 | Centric Tube Pixel 5 Intensity | 000-255 | 0-100% |
| 14 | Centric Tube Pixel 5 Intensity fine | 000-255 | 0-100% |
| 15 | Centric Tube Pixel 6 Intensity | 000-255 | 0-100% |
| 16 | Centric Tube Pixel 6 Intensity fine | 000-255 | 0-100% |
| 17 | Centric Tube Pixel 7 Intensity | 000-255 | 0-100% |
| 18 | Centric Tube Pixel 7 Intensity fine | 000-255 | 0-100% |
| 19 | Centric Tube Pixel 8 Intensity | 000-255 | 0-100% |
| 20 | Centric Tube Pixel 8 Intensity fine | 000-255 | 0-100% |
| 21 | Centric Tube Pixel 9 Intensity | 000-255 | 0-100% |
| 22 | Centric Tube Pixel 9 Intensity fine | 000-255 | 0-100% |
| 23 | Centric Tube Pixel 10 Intensity | 000-255 | 0-100% |
| 24 | Centric Tube Pixel 10 Intensity fine | 000-255 | 0-100% |
| 25 | Centric Tube Pixel 11 Intensity | 000-255 | 0-100% |
| 26 | Centric Tube Pixel 11 Intensity fine | 000-255 | 0-100% |
| 27 | Centric Tube Pixel 12 Intensity | 000-255 | 0-100% |
| 28 | Centric Tube Pixel 12 Intensity fine | 000-255 | 0-100% |
| 29 | Single Tube 1 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 30 | Single Tube 1 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 31 | Single Tube 1 Pixel 1 Intensity | 000-255 | 0-100% |
| 32 | Single Tube 1 Pixel 1 Intensity fine | 000-255 | 0-100% |
| 33 | Single Tube 1 Pixel 2 Intensity | 000-255 | 0-100% |
| 34 | Single Tube 1 Pixel 2 Intensity fine | 000-255 | 0-100% |
| 35 | Single Tube 1 Pixel 3 Intensity | 000-255 | 0-100% |
| 36 | Single Tube 1 Pixel 3 Intensity fine | 000-255 | 0-100% |
| 37 | Single Tube 1 Pixel 4 Intensity | 000-255 | 0-100% |
| 38 | Single Tube 1 Pixel 4 Intensity fine | 000-255 | 0-100% |
| 39 | Single Tube 1 Pixel 5 Intensity | 000-255 | 0-100% |
| 40 | Single Tube 1 Pixel 5 Intensity fine | 000-255 | 0-100% |
| 41 | Single Tube 1 Pixel 6 Intensity | 000-255 | 0-100% |
| 42 | Single Tube 1 Pixel 6 Intensity fine | 000-255 | 0-100% |
| 43 | Single Tube 1 Pixel 7 Intensity | 000-255 | 0-100% |
| 44 | Single Tube 1 Pixel 7 Intensity fine | 000-255 | 0-100% |
| 45 | Single Tube 1 Pixel 8 Intensity | 000-255 | 0-100% |
| 46 | Single Tube 1 Pixel 8 Intensity fine | 000-255 | 0-100% |
| 47 | Single Tube 1 Pixel 9 Intensity | 000-255 | 0-100% |
| 48 | Single Tube 1 Pixel 9 Intensity fine | 000-255 | 0-100% |
| 49 | Single Tube 1 Pixel 10 Intensity | 000-255 | 0-100% |
| 50 | Single Tube 1 Pixel 10 Intensity fine | 000-255 | 0-100% |
| 51 | Single Tube 1 Pixel 11 Intensity | 000-255 | 0-100% |
| 52 | Single Tube 1 Pixel 11 Intensity fine | 000-255 | 0-100% |
| 53 | Single Tube 1 Pixel 12 Intensity | 000-255 | 0-100% |
| 54 | Single Tube 1 Pixel 12 Intensity fine | 000-255 | 0-100% |
| 55 | Single Tube 2 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 56 | Single Tube 2 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 57 | Single Tube 2 Pixel 1 Intensity | 000-255 | 0-100% |
| 58 | Single Tube 2 Pixel 1 Intensity fine | 000-255 | 0-100% |
| 59 | Single Tube 2 Pixel 2 Intensity | 000-255 | 0-100% |
| 60 | Single Tube 2 Pixel 2 Intensity fine | 000-255 | 0-100% |
| 61 | Single Tube 2 Pixel 3 Intensity | 000-255 | 0-100% |
| 62 | Single Tube 2 Pixel 3 Intensity fine | 000-255 | 0-100% |
| 63 | Single Tube 2 Pixel 4 Intensity | 000-255 | 0-100% |
| 64 | Single Tube 2 Pixel 4 Intensity fine | 000-255 | 0-100% |
| 65 | Single Tube 2 Pixel 5 Intensity | 000-255 | 0-100% |
| 66 | Single Tube 2 Pixel 5 Intensity fine | 000-255 | 0-100% |
| 67 | Single Tube 2 Pixel 6 Intensity | 000-255 | 0-100% |
| 68 | Single Tube 2 Pixel 6 Intensity fine | 000-255 | 0-100% |
| 69 | Single Tube 2 Pixel 7 Intensity | 000-255 | 0-100% |
| 70 | Single Tube 2 Pixel 7 Intensity fine | 000-255 | 0-100% |
| 71 | Single Tube 2 Pixel 8 Intensity | 000-255 | 0-100% |
| 72 | Single Tube 2 Pixel 8 Intensity fine | 000-255 | 0-100% |
| 73 | Single Tube 2 Pixel 9 Intensity | 000-255 | 0-100% |
| 74 | Single Tube 2 Pixel 9 Intensity fine | 000-255 | 0-100% |
| 75 | Single Tube 2 Pixel 10 Intensity | 000-255 | 0-100% |
| 76 | Single Tube 2 Pixel 10 Intensity fine | 000-255 | 0-100% |
| 77 | Single Tube 2 Pixel 11 Intensity | 000-255 | 0-100% |
| 78 | Single Tube 2 Pixel 11 Intensity fine | 000-255 | 0-100% |
| 79 | Single Tube 2 Pixel 12 Intensity | 000-255 | 0-100% |
| 80 | Single Tube 2 Pixel 12 Intensity fine | 000-255 | 0-100% |
| 81 | Single Tube 3 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 82 | Single Tube 3 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 83 | Single Tube 3 Pixel 1 Intensity | 000-255 | 0-100% |
| 84 | Single Tube 3 Pixel 1 Intensity fine | 000-255 | 0-100% |
| 85 | Single Tube 3 Pixel 2 Intensity | 000-255 | 0-100% |
| 86 | Single Tube 3 Pixel 2 Intensity fine | 000-255 | 0-100% |
| 87 | Single Tube 3 Pixel 3 Intensity | 000-255 | 0-100% |
| 88 | Single Tube 3 Pixel 3 Intensity fine | 000-255 | 0-100% |
| 89 | Single Tube 3 Pixel 4 Intensity | 000-255 | 0-100% |
| 90 | Single Tube 3 Pixel 4 Intensity fine | 000-255 | 0-100% |
| 91 | Single Tube 3 Pixel 5 Intensity | 000-255 | 0-100% |
| 92 | Single Tube 3 Pixel 5 Intensity fine | 000-255 | 0-100% |
| 93 | Single Tube 3 Pixel 6 Intensity | 000-255 | 0-100% |
| 94 | Single Tube 3 Pixel 6 Intensity fine | 000-255 | 0-100% |
| 95 | Single Tube 3 Pixel 7 Intensity | 000-255 | 0-100% |
| 96 | Single Tube 3 Pixel 7 Intensity fine | 000-255 | 0-100% |
| 97 | Single Tube 3 Pixel 8 Intensity | 000-255 | 0-100% |
| 98 | Single Tube 3 Pixel 8 Intensity fine | 000-255 | 0-100% |
| 99 | Single Tube 3 Pixel 9 Intensity | 000-255 | 0-100% |
| 100 | Single Tube 3 Pixel 9 Intensity fine | 000-255 | 0-100% |
| 101 | Single Tube 3 Pixel 10 Intensity | 000-255 | 0-100% |
| 102 | Single Tube 3 Pixel 10 Intensity fine | 000-255 | 0-100% |
| 103 | Single Tube 3 Pixel 11 Intensity | 000-255 | 0-100% |
| 104 | Single Tube 3 Pixel 11 Intensity fine | 000-255 | 0-100% |
| 105 | Single Tube 3 Pixel 12 Intensity | 000-255 | 0-100% |
| 106 | Single Tube 3 Pixel 12 Intensity fine | 000-255 | 0-100% |
| 107 | Single Tube 4 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 108 | Single Tube 4 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 109 | Single Tube 4 Pixel 1 Intensity | 000-255 | 0-100% |
| 110 | Single Tube 4 Pixel 1 Intensity fine | 000-255 | 0-100% |
| 111 | Single Tube 4 Pixel 2 Intensity | 000-255 | 0-100% |
| 112 | Single Tube 4 Pixel 2 Intensity fine | 000-255 | 0-100% |
| 113 | Single Tube 4 Pixel 3 Intensity | 000-255 | 0-100% |
| 114 | Single Tube 4 Pixel 3 Intensity fine | 000-255 | 0-100% |
| 115 | Single Tube 4 Pixel 4 Intensity | 000-255 | 0-100% |
| 116 | Single Tube 4 Pixel 4 Intensity fine | 000-255 | 0-100% |
| 117 | Single Tube 4 Pixel 5 Intensity | 000-255 | 0-100% |
| 118 | Single Tube 4 Pixel 5 Intensity fine | 000-255 | 0-100% |
| 119 | Single Tube 4 Pixel 6 Intensity | 000-255 | 0-100% |
| 120 | Single Tube 4 Pixel 6 Intensity fine | 000-255 | 0-100% |
| 121 | Single Tube 4 Pixel 7 Intensity | 000-255 | 0-100% |
| 122 | Single Tube 4 Pixel 7 Intensity fine | 000-255 | 0-100% |
| 123 | Single Tube 4 Pixel 8 Intensity | 000-255 | 0-100% |
| 124 | Single Tube 4 Pixel 8 Intensity fine | 000-255 | 0-100% |
| 125 | Single Tube 4 Pixel 9 Intensity | 000-255 | 0-100% |
| 126 | Single Tube 4 Pixel 9 Intensity fine | 000-255 | 0-100% |
| 127 | Single Tube 4 Pixel 10 Intensity | 000-255 | 0-100% |
| 128 | Single Tube 4 Pixel 10 Intensity fine | 000-255 | 0-100% |
| 129 | Single Tube 4 Pixel 11 Intensity | 000-255 | 0-100% |
| 130 | Single Tube 4 Pixel 11 Intensity fine | 000-255 | 0-100% |
| 131 | Single Tube 4 Pixel 12 Intensity | 000-255 | 0-100% |
| 132 | Single Tube 4 Pixel 12 Intensity fine | 000-255 | 0-100% |
| 133 | RGB Panel 1 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 134 | RGB Panel 1 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 135 | RGB Panel 1 Red 1 | 000-255 | 0-100% |
| 136 | RGB Panel 1 Red 1 fine | 000-255 | 0-100% |
| 137 | RGB Panel 1 Green 1 | 000-255 | 0-100% |
| 138 | RGB Panel 1 Green 1 fine | 000-255 | 0-100% |
| 139 | RGB Panel 1 Blue 1 | 000-255 | 0-100% |
| 140 | RGB Panel 1 Blue 1 fine | 000-255 | 0-100% |
| 141 | RGB Panel 1 Red 2 | 000-255 | 0-100% |
| 142 | RGB Panel 1 Red 2 fine | 000-255 | 0-100% |
| 143 | RGB Panel 1 Green 2 | 000-255 | 0-100% |
| 144 | RGB Panel 1 Green 2 fine | 000-255 | 0-100% |
| 145 | RGB Panel 1 Blue 2 | 000-255 | 0-100% |
| 146 | RGB Panel 1 Blue 2 fine | 000-255 | 0-100% |
| 147 | RGB Panel 1 Red 3 | 000-255 | 0-100% |
| 148 | RGB Panel 1 Red 3 fine | 000-255 | 0-100% |
| 149 | RGB Panel 1 Green 3 | 000-255 | 0-100% |
| 150 | RGB Panel 1 Green 3 fine | 000-255 | 0-100% |
| 151 | RGB Panel 1 Blue 3 | 000-255 | 0-100% |
| 152 | RGB Panel 1 Blue 3 fine | 000-255 | 0-100% |
| 153 | RGB Panel 1 Red 4 | 000-255 | 0-100% |
| 154 | RGB Panel 1 Red 4 fine | 000-255 | 0-100% |
| 155 | RGB Panel 1 Green 4 | 000-255 | 0-100% |
| 156 | RGB Panel 1 Green 4 fine | 000-255 | 0-100% |
| 157 | RGB Panel 1 Blue 4 | 000-255 | 0-100% |
| 158 | RGB Panel 1 Blue 4 fine | 000-255 | 0-100% |
| 159 | RGB Panel 1 Red 5 | 000-255 | 0-100% |
| 160 | RGB Panel 1 Red 5 fine | 000-255 | 0-100% |
| 161 | RGB Panel 1 Green 5 | 000-255 | 0-100% |
| 162 | RGB Panel 1 Green 5 fine | 000-255 | 0-100% |
| 163 | RGB Panel 1 Blue 5 | 000-255 | 0-100% |
| 164 | RGB Panel 1 Blue 5 fine | 000-255 | 0-100% |
| 165 | RGB Panel 1 Red 6 | 000-255 | 0-100% |
| 166 | RGB Panel 1 Red 6 fine | 000-255 | 0-100% |
| 167 | RGB Panel 1 Green 6 | 000-255 | 0-100% |
| 168 | RGB Panel 1 Green 6 fine | 000-255 | 0-100% |
| 169 | RGB Panel 1 Blue 6 | 000-255 | 0-100% |
| 170 | RGB Panel 1 Blue 6 fine | 000-255 | 0-100% |
| 171 | RGB Panel 2 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 172 | RGB Panel 2 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 173 | RGB Panel 2 Red 1 | 000-255 | 0-100% |
| 174 | RGB Panel 2 Red 1 fine | 000-255 | 0-100% |
| 175 | RGB Panel 2 Green 1 | 000-255 | 0-100% |
| 176 | RGB Panel 2 Green 1 fine | 000-255 | 0-100% |
| 177 | RGB Panel 2 Blue 1 | 000-255 | 0-100% |
| 178 | RGB Panel 2 Blue 1 fine | 000-255 | 0-100% |
| 179 | RGB Panel 2 Red 2 | 000-255 | 0-100% |
| 180 | RGB Panel 2 Red 2 fine | 000-255 | 0-100% |
| 181 | RGB Panel 2 Green 2 | 000-255 | 0-100% |
| 182 | RGB Panel 2 Green 2 fine | 000-255 | 0-100% |
| 183 | RGB Panel 2 Blue 2 | 000-255 | 0-100% |
| 184 | RGB Panel 2 Blue 2 fine | 000-255 | 0-100% |
| 185 | RGB Panel 2 Red 3 | 000-255 | 0-100% |
| 186 | RGB Panel 2 Red 3 fine | 000-255 | 0-100% |
| 187 | RGB Panel 2 Green 3 | 000-255 | 0-100% |
| 188 | RGB Panel 2 Green 3 fine | 000-255 | 0-100% |
| 189 | RGB Panel 2 Blue 3 | 000-255 | 0-100% |
| 190 | RGB Panel 2 Blue 3 fine | 000-255 | 0-100% |
| 191 | RGB Panel 2 Red 4 | 000-255 | 0-100% |
| 192 | RGB Panel 2 Red 4 fine | 000-255 | 0-100% |
| 193 | RGB Panel 2 Green 4 | 000-255 | 0-100% |
| 194 | RGB Panel 2 Green 4 fine | 000-255 | 0-100% |
| 195 | RGB Panel 2 Blue 4 | 000-255 | 0-100% |
| 196 | RGB Panel 2 Blue 4 fine | 000-255 | 0-100% |
| 197 | RGB Panel 2 Red 5 | 000-255 | 0-100% |
| 198 | RGB Panel 2 Red 5 fine | 000-255 | 0-100% |
| 199 | RGB Panel 2 Green 5 | 000-255 | 0-100% |
| 200 | RGB Panel 2 Green 5 fine | 000-255 | 0-100% |
| 201 | RGB Panel 2 Blue 5 | 000-255 | 0-100% |
| 202 | RGB Panel 2 Blue 5 fine | 000-255 | 0-100% |
| 203 | RGB Panel 2 Red 6 | 000-255 | 0-100% |
| 204 | RGB Panel 2 Red 6 fine | 000-255 | 0-100% |
| 205 | RGB Panel 2 Green 6 | 000-255 | 0-100% |
| 206 | RGB Panel 2 Green 6 fine | 000-255 | 0-100% |
| 207 | RGB Panel 2 Blue 6 | 000-255 | 0-100% |
| 208 | RGB Panel 2 Blue 6 fine | 000-255 | 0-100% |
| 209 | Device Settings (Hold 3s to activate) | 000-019 | No function |
|  |  | 020-024 | Display Backlight On |
|  |  | 025-029 | Display Backlight Off |
|  |  | 105-109 | Fan Auto |
|  |  | 110-114 | Fan Silent |
|  |  | 115-119 | Fan Off |
|  |  | 120-124 | Fan High Power |
|  |  | 130-134 | Invert RGB Panel 1 Off |
|  |  | 135-139 | Invert RGB Panel 1 On |
|  |  | 140-144 | Invert RGB Panel 2 Off |
|  |  | 145-149 | Invert RGB Panel 2 On |
|  |  | 150-154 | Invert Centric Tube Off |
|  |  | 155-159 | Invert Centric Tube On |
|  |  | 160-164 | Invert Centric Tube 1 Off |
|  |  | 165-169 | Invert Centric Tube 1 On |
|  |  | 170-174 | Invert Centric Tube 2 Off |
|  |  | 175-179 | Invert Centric Tube 2 On |
|  |  | 180-184 | Invert Centric Tube 3 Off |
|  |  | 185-189 | Invert Centric Tube 3 On |
|  |  | 190-194 | Invert Centric Tube 4 Off |
|  |  | 195-199 | Invert Centric Tube 4 On |
|  |  | 200-204 | Swap RGB Panels Off |
|  |  | 205-209 | Swap RGB Panels On |
|  |  | 240-244 | Factory Reset |

| 178 FULL ACCESS MODE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CH | Function |  |  | Value |  |  | Remark |  |  |  |  |
| 1 | Master Dimmer |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 2 | Centric Tube Pixel 1 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 3 | Centric Tube Pixel 2 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 4 | Centric Tube Pixel 3 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 5 | Centric Tube Pixel 4 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 6 | Centric Tube Pixel 5 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 7 | Centric Tube Pixel 6 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 8 | Centric Tube Pixel 7 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 9 | Centric Tube Pixel 8 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 10 | Centric Tube Pixel 9 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 11 | Centric Tube Pixel 10 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 12 | Centric Tube Pixel 11 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 13 | Centric Tube Pixel 12 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 14 |  | Centric Tube Strobe |  | 000-019 |  |  | Strobe close |  |  |  |  |
|  |  |  |  | 020-024 |  |  | Strobe open |  |  |  |  |
|  |  |  |  | 025-064 |  |  | Normal strobe (fast->slow) |  |  |  |  |
|  |  |  |  | 065-069 |  |  | Strobe open |  |  |  |  |
|  |  |  |  | 070-084 |  |  | Opening pulse (fast->slow) |  |  |  |  |
|  |  |  |  | 085-089 |  |  | Strobe open |  |  |  |  |
|  |  |  |  | 090-104 |  |  | Closing pulse (fast->slow) |  |  |  |  |
|  |  |  |  | 105-109 |  |  | Strobe open |  |  |  |  |
|  |  |  |  | 110-124 |  |  | Random strobe (fast->slow) |  |  |  |  |
|  |  |  |  | 125-129 |  |  | Strobe open |  |  |  |  |
|  |  |  |  | 130-144 |  |  | Random opening pulse (fast->slow) |  |  |  |  |
|  |  |  |  | 145-149 |  |  | Strobe open |  |  |  |  |
|  |  |  |  | 150-164 |  |  | Random closing pulse (fast->slow) |  |  |  |  |
|  |  |  |  | 165-169 |  |  | Strobe open |  |  |  |  |
|  |  |  |  | 170-184 |  |  | Burst pulse (fast->slow) |  |  |  |  |
|  |  |  |  | 185-189 |  |  | Strobe open |  |  |  |  |
|  |  |  |  | 190-204 |  |  | Random burst pulse (fast->slow) |  |  |  |  |
|  |  |  |  | 205-209 |  |  | Strobe open |  |  |  |  |
|  |  |  |  | 210-224 |  |  | Sine wave (fast->slow) |  |  |  |  |
|  |  |  |  | 225-229 |  |  | Strobe open |  |  |  |  |
|  |  |  |  | 230-244 |  |  | Burst (fast->slow) |  |  |  |  |
|  |  |  |  | 245-255 |  |  | Strobe open |  |  |  |  |
| 15 |  | Centric Tube Duration |  | 000-255 |  |  | Only affects to normal strobe (025-064) |  |  |  |  |
| 16 |  | Centric Tube Pattern Dimmer |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 17 |  | Centric Tube Pattern Selection |  | 000-001 |  |  | No function |  |  |  |  |
|  |  |  |  | 002-143 |  |  | One pattern per value |  |  |  |  |
|  |  |  |  | 144-255 |  |  | No function |  |  |  |  |
| 18 |  | Centric Tube Pattern Selection Fade |  | 000-255 |  |  | Selection fade (fast->slow) |  |  |  |  |
| 19 |  | Centric Tube Pattern Speed |  | 000-124 |  |  | Left to Right / Slow to fast |  |  |  |  |
|  |  |  |  | 125-130 |  |  | No function |  |  |  |  |
|  |  |  |  | 131-249 |  |  | Right to Left / Fast to slow |  |  |  |  |
|  |  |  |  | 250-255 |  |  | No function |  |  |  |  |
| 20 |  | Centric Tube Pattern Fade |  | 000-255 |  |  | Pattern fade (fast->slow) |  |  |  |  |
| 21 |  | Reserve |  | 000-255 |  |  | Reserve |  |  |  |  |
| 22 | Single Tube 1 Pixel 1 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 23 | Single Tube 1 Pixel 2 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 24 | Single Tube 1 Pixel 3 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 25 | Single Tube 1 Pixel 4 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 26 | Single Tube 1 Pixel 5 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 27 | Single Tube 1 Pixel 6 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 28 | Single Tube 1 Pixel 7 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 29 | Single Tube 1 Pixel 8 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 30 | Single Tube 1 Pixel 9 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 31 | Single Tube 1 Pixel 10 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 32 | Single Tube 1 Pixel 11 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 33 | Single Tube 1 Pixel 12 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 34 |  |  | Single Tube 1 Strobe |  |  | 000-019 |  |  | Strobe close |  |
|  |  |  |  |  |  | 020-024 |  |  | Strobe open |  |
|  |  |  |  |  |  | 025-064 |  |  | Normal strobe (fast->slow) |  |
|  |  |  |  |  |  | 065-069 |  |  | Strobe open |  |
|  |  |  |  |  |  | 070-084 |  |  | Opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 085-089 |  |  | Strobe open |  |
|  |  |  |  |  |  | 090-104 |  |  | Closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 105-109 |  |  | Strobe open |  |
|  |  |  |  |  |  | 110-124 |  |  | Random strobe (fast->slow) |  |
|  |  |  |  |  |  | 125-129 |  |  | Strobe open |  |
|  |  |  |  |  |  | 130-144 |  |  | Random opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 145-149 |  |  | Strobe open |  |
|  |  |  |  |  |  | 150-164 |  |  | Random closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 165-169 |  |  | Strobe open |  |
|  |  |  |  |  |  | 170-184 |  |  | Burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 185-189 |  |  | Strobe open |  |
|  |  |  |  |  |  | 190-204 |  |  | Random burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 205-209 |  |  | Strobe open |  |
|  |  |  |  |  |  | 210-224 |  |  | Sine wave (fast->slow) |  |
|  |  |  |  |  |  | 225-229 |  |  | Strobe open |  |
|  |  |  |  |  |  | 230-244 |  |  | Burst (fast->slow) |  |
|  |  |  |  |  |  | 245-255 |  |  | Strobe open |  |
| 35 |  |  | Single Tube 1 Duration |  |  | 000-255 |  |  | Only affects to normal strobe (025-064) |  |
| 36 |  |  | Single Tube 1 Pattern Dimmer |  |  | 000-255 |  |  | 0-100% |  |
| 37 |  |  | Single Tube 1 Pattern Selection |  |  | 000-001 |  |  | No function |  |
|  |  |  |  |  |  | 002-143 |  |  | One pattern per value |  |
|  |  |  |  |  |  | 144-255 |  |  | No function |  |
| 38 |  |  | Single Tube 1 Pattern Selection Fade |  |  | 000-255 |  |  | Selection fade (fast->slow) |  |
| 39 |  |  | Single Tube 1 Pattern Speed |  |  | 000-124 |  |  | Left to Right / Slow to fast |  |
|  |  |  |  |  |  | 125-130 |  |  | No function |  |
|  |  |  |  |  |  | 131-249 |  |  | Right to Left / Fast to slow |  |
|  |  |  |  |  |  | 250-255 |  |  | No function |  |
| 40 |  |  | Single Tube 1 Pattern Fade |  |  | 000-255 |  |  | Pattern fade (fast->slow) |  |
| 41 |  |  | Reserve |  |  | 000-255 |  |  | Reserve |  |
| 42 | Single Tube 2 Pixel 1 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 43 | Single Tube 2 Pixel 2 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 44 | Single Tube 2 Pixel 3 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 45 | Single Tube 2 Pixel 4 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 46 | Single Tube 2 Pixel 5 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 47 | Single Tube 2 Pixel 6 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 48 | Single Tube 2 Pixel 7 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 49 | Single Tube 2 Pixel 8 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 50 | Single Tube 2 Pixel 9 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 51 | Single Tube 2 Pixel 10 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 52 | Single Tube 2 Pixel 11 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 53 | Single Tube 2 Pixel 12 Intensity |  |  | 000-255 |  |  | 0-100% |  |  |  |  |
| 54 |  |  | Single Tube 2 Strobe |  |  | 000-019 |  |  | Strobe close |  |
|  |  |  |  |  |  | 020-024 |  |  | Strobe open |  |
|  |  |  |  |  |  | 025-064 |  |  | Normal strobe (fast->slow) |  |
|  |  |  |  |  |  | 065-069 |  |  | Strobe open |  |
|  |  |  |  |  |  | 070-084 |  |  | Opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 085-089 |  |  | Strobe open |  |
|  |  |  |  |  |  | 090-104 |  |  | Closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 105-109 |  |  | Strobe open |  |
|  |  |  |  |  |  | 110-124 |  |  | Random strobe (fast->slow) |  |
|  |  |  |  |  |  | 125-129 |  |  | Strobe open |  |
|  |  |  |  |  |  | 130-144 |  |  | Random opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 145-149 |  |  | Strobe open |  |
|  |  |  |  |  |  | 150-164 |  |  | Random closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 165-169 |  |  | Strobe open |  |
|  |  |  |  |  |  | 170-184 |  |  | Burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 185-189 |  |  | Strobe open |  |
|  |  |  |  |  |  | 190-204 |  |  | Random burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 205-209 |  |  | Strobe open |  |
|  |  |  |  |  |  | 210-224 |  |  | Sine wave (fast->slow) |  |
|  |  |  |  |  |  | 225-229 |  |  | Strobe open |  |
|  |  |  |  |  |  | 230-244 |  |  | Burst (fast->slow) |  |
|  |  |  |  |  |  | 245-255 |  |  | Strobe open |  |
| 55 |  |  | Single Tube 2 Duration |  |  | 000-255 |  |  | Only affects to normal strobe (025-064) |  |
| 56 |  |  | Single Tube 2 Pattern Dimmer |  |  | 000-255 |  |  | 0-100% |  |
| 57 |  |  | Single Tube 2 Pattern Selection |  |  | 000-001 |  |  | No function |  |
|  |  |  |  |  |  | 002-143 |  |  | One pattern per value |  |
|  |  |  |  |  |  | 144-255 |  |  | No function |  |
| 58 |  |  | Single Tube 2 Pattern Selection Fade |  |  | 000-255 |  |  | Selection fade (fast->slow) |  |
| 59 |  |  | Single Tube 2 Pattern Speed |  |  | 000-124 |  |  | Left to Right / Slow to fast |  |
|  |  |  |  |  |  | 125-130 |  |  | No function |  |
|  |  |  |  |  |  | 131-249 |  |  | Right to Left / Fast to slow |  |
|  |  |  |  |  |  | 250-255 |  |  | No function |  |
| 60 |  |  | Single Tube 2 Pattern Fade |  |  | 000-255 |  |  | Pattern fade (fast->slow) |  |
| 61 |  |  | Reserve |  |  | 000-255 |  |  | Reserve |  |
| 62 |  |  | Single Tube 3 Pixel 1 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 63 |  |  | Single Tube 3 Pixel 2 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 64 |  |  | Single Tube 3 Pixel 3 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 65 |  |  | Single Tube 3 Pixel 4 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 66 |  |  | Single Tube 3 Pixel 5 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 67 |  |  | Single Tube 3 Pixel 6 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 68 |  |  | Single Tube 3 Pixel 7 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 69 |  |  | Single Tube 3 Pixel 8 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 70 |  |  | Single Tube 3 Pixel 9 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 71 |  |  | Single Tube 3 Pixel 10 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 72 |  |  | Single Tube 3 Pixel 11 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 73 |  |  | Single Tube 3 Pixel 12 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 74 |  |  | Single Tube 4 Strobe |  |  | 000-019 |  |  | Strobe close |  |
|  |  |  |  |  |  | 020-024 |  |  | Strobe open |  |
|  |  |  |  |  |  | 025-064 |  |  | Normal strobe (fast->slow) |  |
|  |  |  |  |  |  | 065-069 |  |  | Strobe open |  |
|  |  |  |  |  |  | 070-084 |  |  | Opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 085-089 |  |  | Strobe open |  |
|  |  |  |  |  |  | 090-104 |  |  | Closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 105-109 |  |  | Strobe open |  |
|  |  |  |  |  |  | 110-124 |  |  | Random strobe (fast->slow) |  |
|  |  |  |  |  |  | 125-129 |  |  | Strobe open |  |
|  |  |  |  |  |  | 130-144 |  |  | Random opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 145-149 |  |  | Strobe open |  |
|  |  |  |  |  |  | 150-164 |  |  | Random closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 165-169 |  |  | Strobe open |  |
|  |  |  |  |  |  | 170-184 |  |  | Burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 185-189 |  |  | Strobe open |  |
|  |  |  |  |  |  | 190-204 |  |  | Random burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 205-209 |  |  | Strobe open |  |
|  |  |  |  |  |  | 210-224 |  |  | Sine wave (fast->slow) |  |
|  |  |  |  |  |  | 225-229 |  |  | Strobe open |  |
|  |  |  |  |  |  | 230-244 |  |  | Burst (fast->slow) |  |
|  |  |  |  |  |  | 245-255 |  |  | Strobe open |  |
| 75 |  |  | Single Tube 4 Duration |  |  | 000-255 |  |  | Only affects to normal strobe (025-064) |  |
| 76 |  |  | Single Tube 4 Pattern Dimmer |  |  | 000-255 |  |  | 0-100% |  |
| 77 |  |  | Single Tube 4 Pattern Selection |  |  | 000-001 |  |  | No function |  |
|  |  |  |  |  |  | 002-143 |  |  | One pattern per value |  |
|  |  |  |  |  |  | 144-255 |  |  | No function |  |
| 78 |  |  | Single Tube 4 Pattern Selection Fade |  |  | 000-255 |  |  | Selection fade (fast->slow) |  |
| 79 |  |  | Single Tube 4 Pattern Speed |  |  | 000-124 |  |  | Left to Right / Slow to fast |  |
|  |  |  |  |  |  | 125-130 |  |  | No function |  |
|  |  |  |  |  |  | 131-249 |  |  | Right to Left / Fast to slow |  |
|  |  |  |  |  |  | 250-255 |  |  | No function |  |
| 80 |  |  | Single Tube 4 Pattern Fade |  |  | 000-255 |  |  | Pattern fade (fast->slow) |  |
| 81 |  |  | Reserve |  |  | 000-255 |  |  | Reserve |  |
| 82 |  |  | Single Tube 4 Pixel 1 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 83 |  |  | Single Tube 4 Pixel 2 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 84 |  |  | Single Tube 4 Pixel 3 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 85 |  |  | Single Tube 4 Pixel 4 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 86 |  |  | Single Tube 4 Pixel 5 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 87 |  |  | Single Tube 4 Pixel 6 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 88 |  |  | Single Tube 4 Pixel 7 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 89 |  |  | Single Tube 4 Pixel 8 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 90 |  |  | Single Tube 4 Pixel 9 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 91 |  |  | Single Tube 4 Pixel 10 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 92 |  |  | Single Tube 4 Pixel 11 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 93 |  |  | Single Tube 4 Pixel 12 Intensity |  |  | 000-255 |  |  | 0-100% |  |
| 94 |  |  | Single Tube 4 Strobe |  |  | 000-019 |  |  | Strobe close |  |
|  |  |  |  |  |  | 020-024 |  |  | Strobe open |  |
|  |  |  |  |  |  | 025-064 |  |  | Normal strobe (fast->slow) |  |
|  |  |  |  |  |  | 065-069 |  |  | Strobe open |  |
|  |  |  |  |  |  | 070-084 |  |  | Opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 085-089 |  |  | Strobe open |  |
|  |  |  |  |  |  | 090-104 |  |  | Closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 105-109 |  |  | Strobe open |  |
|  |  |  |  |  |  | 110-124 |  |  | Random strobe (fast->slow) |  |
|  |  |  |  |  |  | 125-129 |  |  | Strobe open |  |
|  |  |  |  |  |  | 130-144 |  |  | Random opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 145-149 |  |  | Strobe open |  |
|  |  |  |  |  |  | 150-164 |  |  | Random closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 165-169 |  |  | Strobe open |  |
|  |  |  |  |  |  | 170-184 |  |  | Burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 185-189 |  |  | Strobe open |  |
|  |  |  |  |  |  | 190-204 |  |  | Random burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 205-209 |  |  | Strobe open |  |
|  |  |  |  |  |  | 210-224 |  |  | Sine wave (fast->slow) |  |
|  |  |  |  |  |  | 225-229 |  |  | Strobe open |  |
|  |  |  |  |  |  | 230-244 |  |  | Burst (fast->slow) |  |
|  |  |  |  |  |  | 245-255 |  |  | Strobe open |  |
| 95 |  |  | Single Tube 4 Duration |  |  | 000-255 |  |  | Only affects to normal strobe (025-064) |  |
| 96 |  |  | Single Tube 4 Pattern Dimmer |  |  | 000-255 |  |  | 0-100% |  |
| 97 |  |  | Single Tube 4 Pattern Selection |  |  | 000-001 |  |  | No function |  |
|  |  |  |  |  |  | 002-143 |  |  | One pattern per value |  |
|  |  |  |  |  |  | 144-255 |  |  | No function |  |
| 98 |  |  | Single Tube 4 Pattern Selection Fade |  |  | 000-255 |  |  | Selection fade (fast->slow) |  |
| 99 |  |  | Single Tube 4 Pattern Speed |  |  | 000-124 |  |  | Left to Right / Slow to fast |  |
|  |  |  |  |  |  | 125-130 |  |  | No function |  |
|  |  |  |  |  |  | 131-249 |  |  | Right to Left / Fast to slow |  |
|  |  |  |  |  |  | 250-255 |  |  | No function |  |
| 100 |  |  | Single Tube 4 Pattern Fade |  |  | 000-255 |  |  | Pattern fade (fast->slow) |  |
| 101 |  |  | Reserve |  |  | 000-255 |  |  | Reserve |  |
| 102 |  |  | Single Tube (1-4) Pattern Dimmer |  |  | 000-255 |  |  | 0-100% |  |
| 103 |  |  | Single Tube (1-4) Pattern Selection |  |  | 000-001 |  |  | No function |  |
|  |  |  |  |  |  | 002-143 |  |  | One pattern per value |  |
|  |  |  |  |  |  | 144-255 |  |  | No function |  |
| 104 |  |  | Single Tube (1-4) Pattern Selection Fade |  |  | 000-255 |  |  | Selection fade (fast->slow) |  |
| 105 |  |  | Single Tube (1-4) Pattern Speed |  |  | 000-124 |  |  | Left to Right / Slow to fast |  |
|  |  |  |  |  |  | 125-130 |  |  | No function |  |
|  |  |  |  |  |  | 131-249 |  |  | Right to Left / Fast to slow |  |
|  |  |  |  |  |  | 250-255 |  |  | No function |  |
| 106 |  |  | Single Tube (1-4) Pattern Fade |  |  | 000-255 |  |  | Pattern fade (fast->slow) |  |
| 107 |  |  | Reserve |  |  | 000-255 |  |  | Reserve |  |
| 108 |  |  | RGB Panel 1 Strobe |  |  | 000-019 |  |  | Strobe close |  |
|  |  |  |  |  |  | 020-024 |  |  | Strobe open |  |
|  |  |  |  |  |  | 025-064 |  |  | Normal strobe (fast->slow) |  |
|  |  |  |  |  |  | 065-069 |  |  | Strobe open |  |
|  |  |  |  |  |  | 070-084 |  |  | Opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 085-089 |  |  | Strobe open |  |
|  |  |  |  |  |  | 090-104 |  |  | Closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 105-109 |  |  | Strobe open |  |
|  |  |  |  |  |  | 110-124 |  |  | Random strobe (fast->slow) |  |
|  |  |  |  |  |  | 125-129 |  |  | Strobe open |  |
|  |  |  |  |  |  | 130-144 |  |  | Random opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 145-149 |  |  | Strobe open |  |
|  |  |  |  |  |  | 150-164 |  |  | Random closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 165-169 |  |  | Strobe open |  |
|  |  |  |  |  |  | 170-184 |  |  | Burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 185-189 |  |  | Strobe open |  |
|  |  |  |  |  |  | 190-204 |  |  | Random burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 205-209 |  |  | Strobe open |  |
|  |  |  |  |  |  | 210-224 |  |  | Sine wave (fast->slow) |  |
|  |  |  |  |  |  | 225-229 |  |  | Strobe open |  |
|  |  |  |  |  |  | 230-244 |  |  | Burst (fast->slow) |  |
|  |  |  |  |  |  | 245-255 |  |  | Strobe open |  |
| 109 |  |  | RGB Panel 1 Duration |  |  | 000-255 |  |  | Only affects to normal strobe (025-064) |  |
| 110 |  |  | RGB Panel 1 Pattern Dimmer |  |  | 000-255 |  |  | 0-100% |  |
| 111 |  |  | RGB Panel 1 Pattern Selection |  |  | 000-001 |  |  | No function |  |
|  |  |  |  |  |  | 002-079 |  |  | One pattern per value |  |
|  |  |  |  |  |  | 080-255 |  |  | No function |  |
| 112 |  |  | RGB Panel 1 Pattern Selection Fade |  |  | 000-255 |  |  | Selection fade (fast->slow) |  |
| 113 |  |  | RGB Panel 1 Pattern Speed |  |  | 000-124 |  |  | Left to Right / Slow to fast |  |
|  |  |  |  |  |  | 125-130 |  |  | No function |  |
|  |  |  |  |  |  | 131-249 |  |  | Right to Left / Fast to slow |  |
|  |  |  |  |  |  | 250-255 |  |  | No function |  |
| 114 |  |  | RGB Panel 1 Pattern Fade |  |  | 000-255 |  |  | Pattern fade (fast->slow) |  |
| 115 |  |  | Reserve |  |  | 000-255 |  |  | Reserve |  |
| 116 |  |  | RGB Panel 1 Red |  |  | 000-255 |  |  | 0-100% |  |
| 117 |  |  | RGB Panel 1 Green |  |  | 000-255 |  |  | 0-100% |  |
| 118 |  |  | RGB Panel 1 Blue |  |  | 000-255 |  |  | 0-100% |  |
| 119 |  |  | RGB Panel 1 CTC |  |  | 000-255 |  |  | CTC |  |
| 120 |  |  | RGB Panel 1 Red 1 |  |  | 000-255 |  |  | 0-100% |  |
| 121 |  |  | RGB Panel 1 Green 1 |  |  | 000-255 |  |  | 0-100% |  |
| 122 |  |  | RGB Panel 1 Blue 1 |  |  | 000-255 |  |  | 0-100% |  |
| 123 |  |  | RGB Panel 1 Red 2 |  |  | 000-255 |  |  | 0-100% |  |
| 124 |  |  | RGB Panel 1 Green 2 |  |  | 000-255 |  |  | 0-100% |  |
| 125 |  |  | RGB Panel 1 Blue 2 |  |  | 000-255 |  |  | 0-100% |  |
| 126 |  |  | RGB Panel 1 Red 3 |  |  | 000-255 |  |  | 0-100% |  |
| 127 |  |  | RGB Panel 1 Green 3 |  |  | 000-255 |  |  | 0-100% |  |
| 128 |  |  | RGB Panel 1 Blue 3 |  |  | 000-255 |  |  | 0-100% |  |
| 129 |  |  | RGB Panel 1 Red 4 |  |  | 000-255 |  |  | 0-100% |  |
| 130 |  |  | RGB Panel 1 Green 4 |  |  | 000-255 |  |  | 0-100% |  |
| 131 |  |  | RGB Panel 1 Blue 4 |  |  | 000-255 |  |  | 0-100% |  |
| 132 |  |  | RGB Panel 1 Red 5 |  |  | 000-255 |  |  | 0-100% |  |
| 133 |  |  | RGB Panel 1 Green 5 |  |  | 000-255 |  |  | 0-100% |  |
| 134 |  |  | RGB Panel 1 Blue 5 |  |  | 000-255 |  |  | 0-100% |  |
| 135 |  |  | RGB Panel 1 Red 6 |  |  | 000-255 |  |  | 0-100% |  |
| 136 |  |  | RGB Panel 1 Green 6 |  |  | 000-255 |  |  | 0-100% |  |
| 137 |  |  | RGB Panel 1 Blue 6 |  |  | 000-255 |  |  | 0-100% |  |
| 138 |  |  | RGB Panel 2 Strobe |  |  | 000-019 |  |  | Strobe close |  |
|  |  |  |  |  |  | 020-024 |  |  | Strobe open |  |
|  |  |  |  |  |  | 025-064 |  |  | Normal strobe (fast->slow) |  |
|  |  |  |  |  |  | 065-069 |  |  | Strobe open |  |
|  |  |  |  |  |  | 070-084 |  |  | Opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 085-089 |  |  | Strobe open |  |
|  |  |  |  |  |  | 090-104 |  |  | Closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 105-109 |  |  | Strobe open |  |
|  |  |  |  |  |  | 110-124 |  |  | Random strobe (fast->slow) |  |
|  |  |  |  |  |  | 125-129 |  |  | Strobe open |  |
|  |  |  |  |  |  | 130-144 |  |  | Random opening pulse (fast->slow) |  |
|  |  |  |  |  |  | 145-149 |  |  | Strobe open |  |
|  |  |  |  |  |  | 150-164 |  |  | Random closing pulse (fast->slow) |  |
|  |  |  |  |  |  | 165-169 |  |  | Strobe open |  |
|  |  |  |  |  |  | 170-184 |  |  | Burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 185-189 |  |  | Strobe open |  |
|  |  |  |  |  |  | 190-204 |  |  | Random burst pulse (fast->slow) |  |
|  |  |  |  |  |  | 205-209 |  |  | Strobe open |  |
|  |  |  |  |  |  | 210-224 |  |  | Sine wave (fast->slow) |  |
|  |  |  |  |  |  | 225-229 |  |  | Strobe open |  |
|  |  |  |  |  |  | 230-244 |  |  | Burst (fast->slow) |  |
|  |  |  |  |  |  | 245-255 |  |  | Strobe open |  |
| 139 |  |  | RGB Panel 2 Duration |  |  | 000-255 |  |  | Only affects to normal strobe (025-064) |  |
| 140 |  |  | RGB Panel 2 Pattern Dimmer |  |  | 000-255 |  |  | 0-100% |  |
| 141 |  |  | RGB Panel 2 Pattern Selection |  |  | 000-001 |  |  | No function |  |
|  |  |  |  |  |  | 002-079 |  |  | One pattern per value |  |
|  |  |  |  |  |  | 080-255 |  |  | No function |  |
| 142 |  |  | RGB Panel 2 Pattern Selection Fade |  |  | 000-255 |  |  | Selection fade (fast->slow) |  |
| 143 |  |  | RGB Panel 2 Pattern Speed |  |  | 000-124 |  |  | Left to Right / Slow to fast |  |
|  |  |  |  |  |  | 125-130 |  |  | No function |  |
|  |  |  |  |  |  | 131-249 |  |  | Right to Left / Fast to slow |  |
|  |  |  |  |  |  | 250-255 |  |  | No function |  |
| 144 |  |  | RGB Panel 2 Pattern Fade |  |  | 000-255 |  |  | Pattern fade (fast->slow) |  |
| 145 |  |  | Reserve |  |  | 000-255 |  |  | Reserve |  |
| 146 |  |  | RGB Panel 2 Red |  |  | 000-255 |  |  | 0-100% |  |
| 147 |  |  | RGB Panel 2 Green |  |  | 000-255 |  |  | 0-100% |  |
| 148 |  |  | RGB Panel 2 Blue |  |  | 000-255 |  |  | 0-100% |  |
| 149 |  |  | RGB Panel 2 CTC |  |  | 000-255 |  |  | CTC |  |
| 150 |  |  | RGB Panel 2 Red 1 |  |  | 000-255 |  |  | 0-100% |  |
| 151 |  |  | RGB Panel 2 Green 1 |  |  | 000-255 |  |  | 0-100% |  |
| 152 |  |  | RGB Panel 2 Blue 1 |  |  | 000-255 |  |  | 0-100% |  |
| 153 |  |  | RGB Panel 2 Red 2 |  |  | 000-255 |  |  | 0-100% |  |
| 154 |  |  | RGB Panel 2 Green 2 |  |  | 000-255 |  |  | 0-100% |  |
| 155 |  |  | RGB Panel 2 Blue 2 |  |  | 000-255 |  |  | 0-100% |  |
| 156 |  |  | RGB Panel 2 Red 3 |  |  | 000-255 |  |  | 0-100% |  |
| 157 |  |  | RGB Panel 2 Green 3 |  |  | 000-255 |  |  | 0-100% |  |
| 158 |  |  | RGB Panel 2 Blue 3 |  |  | 000-255 |  |  | 0-100% |  |
| 159 |  |  | RGB Panel 2 Red 4 |  |  | 000-255 |  |  | 0-100% |  |
| 160 |  |  | RGB Panel 2 Green 4 |  |  | 000-255 |  |  | 0-100% |  |
| 161 |  |  | RGB Panel 2 Blue 4 |  |  | 000-255 |  |  | 0-100% |  |
| 162 |  |  | RGB Panel 2 Red 5 |  |  | 000-255 |  |  | 0-100% |  |
| 163 |  |  | RGB Panel 2 Green 5 |  |  | 000-255 |  |  | 0-100% |  |
| 164 |  |  | RGB Panel 2 Blue 5 |  |  | 000-255 |  |  | 0-100% |  |
| 165 |  |  | RGB Panel 2 Red 6 |  |  | 000-255 |  |  | 0-100% |  |
| 166 |  |  | RGB Panel 2 Green 6 |  |  | 000-255 |  |  | 0-100% |  |
| 167 |  |  | RGB Panel 2 Blue 6 |  |  | 000-255 |  |  | 0-100% |  |
| 168 |  |  | RGB Panels (1+2) Pattern Dimmer |  |  | 000-255 |  |  | 0-100% |  |
| 169 |  |  | RGB Panels (1+2) Pattern Selection |  |  | 000-001 |  |  | No function |  |
|  |  |  |  |  |  | 002-079 |  |  | One pattern per value |  |
|  |  |  |  |  |  | 080-255 |  |  | No function |  |
| 170 |  |  | RGB Panels (1+2) Pattern Selection Fade |  |  | 000-255 |  |  | Selection fade (fast->slow) |  |
| 171 |  |  | RGB Panels (1+2) Pattern Speed |  |  | 000-124 |  |  | Left to Right / Slow to fast |  |
|  |  |  |  |  |  | 125-130 |  |  | No function |  |
|  |  |  |  |  |  | 131-249 |  |  | Right to Left / Fast to slow |  |
|  |  |  |  |  |  | 250-255 |  |  | No function |  |
| 172 |  |  | RGB Panels (1+2) Pattern Fade |  |  | 000-255 |  |  | Pattern fade (fast->slow) |  |
| 173 |  |  | Reserve |  |  | 000-255 |  |  | Reserve |  |
| 174 |  |  | RGB Panels (1+2) Pattern Red |  |  | 000-255 |  |  | 0-100% |  |
| 175 |  |  | RGB Panels (1+2) Pattern Green |  |  | 000-255 |  |  | 0-100% |  |
| 176 |  |  | RGB Panels (1+2) Pattern Blue |  |  | 000-255 |  |  | 0-100% |  |
| 177 |  |  | RGB Panels (1+2) Pattern CTC |  |  | 000-255 |  |  | Pattern CTC |  |
| 178 |  |  | Device Settings (Hold 3s to activate) |  |  | 000-019 |  |  | No function |  |
|  |  |  |  |  |  | 020-024 |  |  | Display Backlight On |  |
|  |  |  |  |  |  | 025-029 |  |  | Display Backlight Off |  |
|  |  |  |  |  |  | 105-109 |  |  | Fan Auto |  |
|  |  |  |  |  |  | 110-114 |  |  | Fan Silent |  |
|  |  |  |  |  |  | 115-119 |  |  | Fan Off |  |
|  |  |  |  |  |  | 120-124 |  |  | Fan High Power |  |
|  |  |  |  |  |  | 130-134 |  |  | Invert RGB Panel 1 Off |  |
|  |  |  |  |  |  | 135-139 |  |  | Invert RGB Panel 1 On |  |
|  |  |  |  |  |  | 140-144 |  |  | Invert RGB Panel 2 Off |  |
|  |  |  |  |  |  | 145-149 |  |  | Invert RGB Panel 2 On |  |
|  |  |  |  |  |  | 150-154 |  |  | Invert Centric Tube Off |  |
|  |  |  |  |  |  | 155-159 |  |  | Invert Centric Tube On |  |
|  |  |  |  |  |  | 160-164 |  |  | Invert Centric Tube 1 Off |  |
|  |  |  |  |  |  | 165-169 |  |  | Invert Centric Tube 1 On |  |
|  |  |  |  |  |  | 170-174 |  |  | Invert Centric Tube 2 Off |  |
|  |  |  |  |  |  | 175-179 |  |  | Invert Centric Tube 2 On |  |
|  |  |  |  |  |  | 180-184 |  |  | Invert Centric Tube 3 Off |  |
|  |  |  |  |  |  | 185-189 |  |  | Invert Centric Tube 3 On |  |
|  |  |  |  |  |  | 190-194 |  |  | Invert Centric Tube 4 Off |  |
|  |  |  |  |  |  | 195-199 |  |  | Invert Centric Tube 4 On |  |
|  |  |  |  |  |  | 200-204 |  |  | Swap RGB Panels Off |  |
|  |  |  |  |  |  | 205-209 |  |  | Swap RGB Panels On |  |
|  |  |  |  |  |  | 240-244 |  |  | Factory Reset |  |

| 275CH FULL ACCESS EXTEND MODE |  |  |  |
| --- | --- | --- | --- |
| CH | Function | Value | Remark |
| 1 | Master Dimmer | 000-255 | 0-100% |
| 2 | Master Dimmer fine | 000-255 | 0-100% |
| 3 | Centric Tube Pixel 1 Intensity | 000-255 | 0-100% |
| 4 | Centric Tube Pixel 1 Intensity fine | 000-255 | 0-100% |
| 5 | Centric Tube Pixel 2 Intensity | 000-255 | 0-100% |
| 6 | Centric Tube Pixel 2 Intensity fine | 000-255 | 0-100% |
| 7 | Centric Tube Pixel 3 Intensity | 000-255 | 0-100% |
| 8 | Centric Tube Pixel 3 Intensity fine | 000-255 | 0-100% |
| 9 | Centric Tube Pixel 4 Intensity | 000-255 | 0-100% |
| 10 | Centric Tube Pixel 4 Intensity fine | 000-255 | 0-100% |
| 11 | Centric Tube Pixel 5 Intensity | 000-255 | 0-100% |
| 12 | Centric Tube Pixel 5 Intensity fine | 000-255 | 0-100% |
| 13 | Centric Tube Pixel 6 Intensity | 000-255 | 0-100% |
| 14 | Centric Tube Pixel 6 Intensity fine | 000-255 | 0-100% |
| 15 | Centric Tube Pixel 7 Intensity | 000-255 | 0-100% |
| 16 | Centric Tube Pixel 7 Intensity fine | 000-255 | 0-100% |
| 17 | Centric Tube Pixel 8 Intensity | 000-255 | 0-100% |
| 18 | Centric Tube Pixel 8 Intensity fine | 000-255 | 0-100% |
| 19 | Centric Tube Pixel 9 Intensity | 000-255 | 0-100% |
| 20 | Centric Tube Pixel 9 Intensity fine | 000-255 | 0-100% |
| 21 | Centric Tube Pixel 10 Intensity | 000-255 | 0-100% |
| 22 | Centric Tube Pixel 10 Intensity fine | 000-255 | 0-100% |
| 23 | Centric Tube Pixel 11 Intensity | 000-255 | 0-100% |
| 24 | Centric Tube Pixel 11 Intensity fine | 000-255 | 0-100% |
| 25 | Centric Tube Pixel 12 Intensity | 000-255 | 0-100% |
| 26 | Centric Tube Pixel 12 Intensity fine | 000-255 | 0-100% |
| 27 | Centric Tube Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 28 | Centric Tube Duration | 000-255 | Only affects to normal strobe (025-064) |
| 29 | Centric Tube Pattern Dimmer | 000-255 | 0-100% |
| 30 | Centric Tube Pattern Selection | 000-001 | No function |
|  |  | 002-143 | One pattern per value |
|  |  | 144-255 | No function |
| 31 | Centric Tube Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 32 | Centric Tube Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 33 | Centric Tube Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 34 | Reserve | 000-255 | Reserve |
| 35 | Single Tube 1 Pixel 1 Intensity | 000-255 | 0-100% |
| 36 | Single Tube 1 Pixel 1 Intensity fine | 000-255 | 0-100% |
| 37 | Single Tube 1 Pixel 2 Intensity | 000-255 | 0-100% |
| 38 | Single Tube 1 Pixel 2 Intensity fine | 000-255 | 0-100% |
| 39 | Single Tube 1 Pixel 3 Intensity | 000-255 | 0-100% |
| 40 | Single Tube 1 Pixel 3 Intensity fine | 000-255 | 0-100% |
| 41 | Single Tube 1 Pixel 4 Intensity | 000-255 | 0-100% |
| 42 | Single Tube 1 Pixel 4 Intensity fine | 000-255 | 0-100% |
| 43 | Single Tube 1 Pixel 5 Intensity | 000-255 | 0-100% |
| 44 | Single Tube 1 Pixel 5 Intensity fine | 000-255 | 0-100% |
| 45 | Single Tube 1 Pixel 6 Intensity | 000-255 | 0-100% |
| 46 | Single Tube 1 Pixel 6 Intensity fine | 000-255 | 0-100% |
| 47 | Single Tube 1 Pixel 7 Intensity | 000-255 | 0-100% |
| 48 | Single Tube 1 Pixel 7 Intensity fine | 000-255 | 0-100% |
| 49 | Single Tube 1 Pixel 8 Intensity | 000-255 | 0-100% |
| 50 | Single Tube 1 Pixel 8 Intensity fine | 000-255 | 0-100% |
| 51 | Single Tube 1 Pixel 9 Intensity | 000-255 | 0-100% |
| 52 | Single Tube 1 Pixel 9 Intensity fine | 000-255 | 0-100% |
| 53 | Single Tube 1 Pixel 10 Intensity | 000-255 | 0-100% |
| 54 | Single Tube 1 Pixel 10 Intensity fine | 000-255 | 0-100% |
| 55 | Single Tube 1 Pixel 11 Intensity | 000-255 | 0-100% |
| 56 | Single Tube 1 Pixel 11 Intensity fine | 000-255 | 0-100% |
| 57 | Single Tube 1 Pixel 12 Intensity | 000-255 | 0-100% |
| 58 | Single Tube 1 Pixel 12 Intensity fine | 000-255 | 0-100% |
| 59 | Single Tube 1 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 60 | Single Tube 1 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 61 |  | Single Tube 1 Pattern Dimmer | 000-255 | 0-100% |
| 62 |  | Single Tube 1 Pattern Selection | 000-001 | No function |
|  |  |  | 002-143 | One pattern per value |
|  |  |  | 144-255 | No function |
| 63 |  | Single Tube 1 Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 64 |  | Single Tube 1 Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  |  | 125-130 | No function |
|  |  |  | 131-249 | Right to Left / Fast to slow |
|  |  |  | 250-255 | No function |
| 65 |  | Single Tube 1 Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 66 |  | Reserve | 000-255 | Reserve |
| 67 | Single Tube 2 Pixel 1 Intensity | 000-255 | 0-100% |
| 68 | Single Tube 2 Pixel 1 Intensity fine | 000-255 | 0-100% |
| 69 | Single Tube 2 Pixel 2 Intensity | 000-255 | 0-100% |
| 70 | Single Tube 2 Pixel 2 Intensity fine | 000-255 | 0-100% |
| 71 | Single Tube 2 Pixel 3 Intensity | 000-255 | 0-100% |
| 72 | Single Tube 2 Pixel 3 Intensity fine | 000-255 | 0-100% |
| 73 | Single Tube 2 Pixel 4 Intensity | 000-255 | 0-100% |
| 74 | Single Tube 2 Pixel 4 Intensity fine | 000-255 | 0-100% |
| 75 | Single Tube 2 Pixel 5 Intensity | 000-255 | 0-100% |
| 76 | Single Tube 2 Pixel 5 Intensity fine | 000-255 | 0-100% |
| 77 | Single Tube 2 Pixel 6 Intensity | 000-255 | 0-100% |
| 78 | Single Tube 2 Pixel 6 Intensity fine | 000-255 | 0-100% |
| 79 | Single Tube 2 Pixel 7 Intensity | 000-255 | 0-100% |
| 80 | Single Tube 2 Pixel 7 Intensity fine | 000-255 | 0-100% |
| 81 | Single Tube 2 Pixel 8 Intensity | 000-255 | 0-100% |
| 82 | Single Tube 2 Pixel 8 Intensity fine | 000-255 | 0-100% |
| 83 | Single Tube 2 Pixel 9 Intensity | 000-255 | 0-100% |
| 84 | Single Tube 2 Pixel 9 Intensity fine | 000-255 | 0-100% |
| 85 | Single Tube 2 Pixel 10 Intensity | 000-255 | 0-100% |
| 86 | Single Tube 2 Pixel 10 Intensity fine | 000-255 | 0-100% |
| 87 | Single Tube 2 Pixel 11 Intensity | 000-255 | 0-100% |
| 88 | Single Tube 2 Pixel 11 Intensity fine | 000-255 | 0-100% |
| 89 | Single Tube 2 Pixel 12 Intensity | 000-255 | 0-100% |
| 90 | Single Tube 2 Pixel 12 Intensity fine | 000-255 | 0-100% |
| 91 | Single Tube 2 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 92 | Single Tube 2 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 93 | Single Tube 2 Pattern Dimmer | 000-255 | 0-100% |
| 94 | Single Tube 2 Pattern Selection | 000-001 | No function |
|  |  | 002-143 | One pattern per value |
|  |  | 144-255 | No function |
| 95 | Single Tube 2 Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 96 | Single Tube 2 Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 97 | Single Tube 2 Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 98 | Reserve | 000-255 | Reserve |
| 99 | Single Tube 3 Pixel 1 Intensity | 000-255 | 0-100% |
| 100 | Single Tube 3 Pixel 1 Intensity fine | 000-255 | 0-100% |
| 101 | Single Tube 3 Pixel 2 Intensity | 000-255 | 0-100% |
| 102 | Single Tube 3 Pixel 2 Intensity fine | 000-255 | 0-100% |
| 103 | Single Tube 3 Pixel 3 Intensity | 000-255 | 0-100% |
| 104 | Single Tube 3 Pixel 3 Intensity fine | 000-255 | 0-100% |
| 105 | Single Tube 3 Pixel 4 Intensity | 000-255 | 0-100% |
| 106 | Single Tube 3 Pixel 4 Intensity fine | 000-255 | 0-100% |
| 107 | Single Tube 3 Pixel 5 Intensity | 000-255 | 0-100% |
| 108 | Single Tube 3 Pixel 5 Intensity fine | 000-255 | 0-100% |
| 109 | Single Tube 3 Pixel 6 Intensity | 000-255 | 0-100% |
| 110 | Single Tube 3 Pixel 6 Intensity fine | 000-255 | 0-100% |
| 111 | Single Tube 3 Pixel 7 Intensity | 000-255 | 0-100% |
| 112 | Single Tube 3 Pixel 7 Intensity fine | 000-255 | 0-100% |
| 113 | Single Tube 3 Pixel 8 Intensity | 000-255 | 0-100% |
| 114 | Single Tube 3 Pixel 8 Intensity fine | 000-255 | 0-100% |
| 115 | Single Tube 3 Pixel 9 Intensity | 000-255 | 0-100% |
| 116 | Single Tube 3 Pixel 9 Intensity fine | 000-255 | 0-100% |
| 117 | Single Tube 3 Pixel 10 Intensity | 000-255 | 0-100% |
| 118 | Single Tube 3 Pixel 10 Intensity fine | 000-255 | 0-100% |
| 119 | Single Tube 3 Pixel 11 Intensity | 000-255 | 0-100% |
| 120 | Single Tube 3 Pixel 11 Intensity fine | 000-255 | 0-100% |
| 121 | Single Tube 3 Pixel 12 Intensity | 000-255 | 0-100% |
| 122 | Single Tube 3 Pixel 12 Intensity fine | 000-255 | 0-100% |
| 123 | Single Tube 3 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 124 | Single Tube 3 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 125 |  | Single Tube 3 Pattern Dimmer | 000-255 | 0-100% |
| 126 |  | Single Tube 3 Pattern Selection | 000-001 | No function |
|  |  |  | 002-143 | One pattern per value |
|  |  |  | 144-255 | No function |
| 127 |  | Single Tube 3 Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 128 |  | Single Tube 3 Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  |  | 125-130 | No function |
|  |  |  | 131-249 | Right to Left / Fast to slow |
|  |  |  | 250-255 | No function |
| 129 |  | Single Tube 3 Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 130 |  | Reserve | 000-255 | Reserve |
| 131 | Single Tube 4 Pixel 1 Intensity | 000-255 | 0-100% |
| 132 | Single Tube 4 Pixel 1 Intensity fine | 000-255 | 0-100% |
| 133 | Single Tube 4 Pixel 2 Intensity | 000-255 | 0-100% |
| 134 | Single Tube 4 Pixel 2 Intensity fine | 000-255 | 0-100% |
| 135 | Single Tube 4 Pixel 3 Intensity | 000-255 | 0-100% |
| 136 | Single Tube 4 Pixel 3 Intensity fine | 000-255 | 0-100% |
| 137 | Single Tube 4 Pixel 4 Intensity | 000-255 | 0-100% |
| 138 | Single Tube 4 Pixel 4 Intensity fine | 000-255 | 0-100% |
| 139 | Single Tube 4 Pixel 5 Intensity | 000-255 | 0-100% |
| 140 | Single Tube 4 Pixel 5 Intensity fine | 000-255 | 0-100% |
| 141 | Single Tube 4 Pixel 6 Intensity | 000-255 | 0-100% |
| 142 | Single Tube 4 Pixel 6 Intensity fine | 000-255 | 0-100% |
| 143 | Single Tube 4 Pixel 7 Intensity | 000-255 | 0-100% |
| 144 | Single Tube 4 Pixel 7 Intensity fine | 000-255 | 0-100% |
| 145 | Single Tube 4 Pixel 8 Intensity | 000-255 | 0-100% |
| 146 | Single Tube 4 Pixel 8 Intensity fine | 000-255 | 0-100% |
| 147 | Single Tube 4 Pixel 9 Intensity | 000-255 | 0-100% |
| 148 | Single Tube 4 Pixel 9 Intensity fine | 000-255 | 0-100% |
| 149 | Single Tube 4 Pixel 10 Intensity | 000-255 | 0-100% |
| 150 | Single Tube 4 Pixel 10 Intensity fine | 000-255 | 0-100% |
| 151 | Single Tube 4 Pixel 11 Intensity | 000-255 | 0-100% |
| 152 | Single Tube 4 Pixel 11 Intensity fine | 000-255 | 0-100% |
| 153 | Single Tube 4 Pixel 12 Intensity | 000-255 | 0-100% |
| 154 | Single Tube 4 Pixel 12 Intensity fine | 000-255 | 0-100% |
| 155 | Single Tube 4 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 156 | Single Tube 4 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 157 |  | Single Tube 4 Pattern Dimmer | 000-255 | 0-100% |
| 158 |  | Single Tube 4 Pattern Selection | 000-001 | No function |
|  |  |  | 002-143 | One pattern per value |
|  |  |  | 144-255 | No function |
| 159 |  | Single Tube 4 Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 160 |  | Single Tube 4 Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  |  | 125-130 | No function |
|  |  |  | 131-249 | Right to Left / Fast to slow |
|  |  |  | 250-255 | No function |
| 161 |  | Single Tube 4 Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 162 |  | Reserve | 000-255 | Reserve |
| 163 |  | Single Tube (1-4) Pattern Dimmer | 000-255 | 0-100% |
| 164 |  | Single Tube (1-4) Pattern Selection | 000-001 | No function |
|  |  |  | 002-143 | One pattern per value |
|  |  |  | 144-255 | No function |
| 165 |  | Single Tube (1-4) Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 166 |  | Single Tube (1-4) Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  |  | 125-130 | No function |
|  |  |  | 131-249 | Right to Left / Fast to slow |
|  |  |  | 250-255 | No function |
| 167 |  | Single Tube (1-4) Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 168 |  | Reserve | 000-255 | Reserve |
| 169 | RGB Panel 1 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 170 | RGB Panel 1 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 171 | RGB Panel 1 Pattern Dimmer | 000-255 | 0-100% |
| 172 | RGB Panel 1 Pattern Selection | 000-001 | No function |
|  |  | 002-079 | One pattern per value |
|  |  | 080-255 | No function |
| 173 | RGB Panel 1 Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 174 | RGB Panel 1 Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 175 | RGB Panel 1 Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 176 | Reserve | 000-255 | Reserve |
| 177 | RGB Panel 1 Pattern Red | 000-255 | 0-100% |
| 178 | RGB Panel 1 Pattern Green | 000-255 | 0-100% |
| 179 | RGB Panel 1 Pattern Blue | 000-255 | 0-100% |
| 180 | RGB Panel 1 Pattern CTC | 000-255 | Pattern CTC |
| 181 | RGB Panel 1 Red 1 | 000-255 | 0-100% |
| 182 | RGB Panel 1 Red 1 fine | 000-255 | 0-100% |
| 183 | RGB Panel 1 Green 1 | 000-255 | 0-100% |
| 184 | RGB Panel 1 Green 1 fine | 000-255 | 0-100% |
| 185 | RGB Panel 1 Blue 1 | 000-255 | 0-100% |
| 186 | RGB Panel 1 Blue 1 fine | 000-255 | 0-100% |
| 187 | RGB Panel 1 Red 2 | 000-255 | 0-100% |
| 188 | RGB Panel 1 Red 2 fine | 000-255 | 0-100% |
| 189 | RGB Panel 1 Green 2 | 000-255 | 0-100% |
| 190 | RGB Panel 1 Green 2 fine | 000-255 | 0-100% |
| 191 | RGB Panel 1 Blue 2 | 000-255 | 0-100% |
| 192 | RGB Panel 1 Blue 2 fine | 000-255 | 0-100% |
| 193 | RGB Panel 1 Red 3 | 000-255 | 0-100% |
| 194 | RGB Panel 1 Red 3 fine | 000-255 | 0-100% |
| 195 | RGB Panel 1 Green 3 | 000-255 | 0-100% |
| 196 | RGB Panel 1 Green 3 fine | 000-255 | 0-100% |
| 197 | RGB Panel 1 Blue 3 | 000-255 | 0-100% |
| 198 | RGB Panel 1 Blue 3 fine | 000-255 | 0-100% |
| 199 | RGB Panel 1 Red 4 | 000-255 | 0-100% |
| 200 | RGB Panel 1 Red 4 fine | 000-255 | 0-100% |
| 201 | RGB Panel 1 Green 4 | 000-255 | 0-100% |
| 202 | RGB Panel 1 Green 4 fine | 000-255 | 0-100% |
| 203 | RGB Panel 1 Blue 4 | 000-255 | 0-100% |
| 204 | RGB Panel 1 Blue 4 fine | 000-255 | 0-100% |
| 205 | RGB Panel 1 Red 5 | 000-255 | 0-100% |
| 206 | RGB Panel 1 Red 5 fine | 000-255 | 0-100% |
| 207 | RGB Panel 1 Green 5 | 000-255 | 0-100% |
| 208 | RGB Panel 1 Green 5 fine | 000-255 | 0-100% |
| 209 | RGB Panel 1 Blue 5 | 000-255 | 0-100% |
| 210 | RGB Panel 1 Blue 5 fine | 000-255 | 0-100% |
| 211 | RGB Panel 1 Red 6 | 000-255 | 0-100% |
| 212 | RGB Panel 1 Red 6 fine | 000-255 | 0-100% |
| 213 | RGB Panel 1 Green 6 | 000-255 | 0-100% |
| 214 | RGB Panel 1 Green 6 fine | 000-255 | 0-100% |
| 215 | RGB Panel 1 Blue 6 | 000-255 | 0-100% |
| 216 | RGB Panel 1 Blue 6 fine | 000-255 | 0-100% |
| 217 | RGB Panel 2 Strobe | 000-019 | Strobe close |
|  |  | 020-024 | Strobe open |
|  |  | 025-064 | Normal strobe (fast->slow) |
|  |  | 065-069 | Strobe open |
|  |  | 070-084 | Opening pulse (fast->slow) |
|  |  | 085-089 | Strobe open |
|  |  | 090-104 | Closing pulse (fast->slow) |
|  |  | 105-109 | Strobe open |
|  |  | 110-124 | Random strobe (fast->slow) |
|  |  | 125-129 | Strobe open |
|  |  | 130-144 | Random opening pulse (fast->slow) |
|  |  | 145-149 | Strobe open |
|  |  | 150-164 | Random closing pulse (fast->slow) |
|  |  | 165-169 | Strobe open |
|  |  | 170-184 | Burst pulse (fast->slow) |
|  |  | 185-189 | Strobe open |
|  |  | 190-204 | Random burst pulse (fast->slow) |
|  |  | 205-209 | Strobe open |
|  |  | 210-224 | Sine wave (fast->slow) |
|  |  | 225-229 | Strobe open |
|  |  | 230-244 | Burst (fast->slow) |
|  |  | 245-255 | Strobe open |
| 218 | RGB Panel 2 Duration | 000-255 | Only affects to normal strobe (025-064) |
| 219 | RGB Panel 2 Pattern Dimmer | 000-255 | 0-100% |
| 220 | RGB Panel 2 Pattern Selection | 000-001 | No function |
|  |  | 002-079 | One pattern per value |
|  |  | 080-255 | No function |
| 221 | RGB Panel 2 Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 222 | RGB Panel 2 Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  | 125-130 | No function |
|  |  | 131-249 | Right to Left / Fast to slow |
|  |  | 250-255 | No function |
| 223 | RGB Panel 2 Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 224 | Reserve | 000-255 | Reserve |
| 225 | RGB Panel 2 Pattern Red | 000-255 | 0-100% |
| 226 | RGB Panel 2 Pattern Green | 000-255 | 0-100% |
| 227 | RGB Panel 2 Pattern Blue | 000-255 | 0-100% |
| 228 | RGB Panel 2 Pattern CTC | 000-255 | Pattern CTC |
| 229 | RGB Panel 2 Red 1 | 000-255 | 0-100% |
| 230 | RGB Panel 2 Red 1 fine | 000-255 | 0-100% |
| 231 | RGB Panel 2 Green 1 | 000-255 | 0-100% |
| 232 | RGB Panel 2 Green 1 fine | 000-255 | 0-100% |
| 233 | RGB Panel 2 Blue 1 | 000-255 | 0-100% |
| 234 | RGB Panel 2 Blue 1 fine | 000-255 | 0-100% |
| 235 | RGB Panel 2 Red 2 | 000-255 | 0-100% |
| 236 | RGB Panel 2 Red 2 fine | 000-255 | 0-100% |
| 237 | RGB Panel 2 Green 2 | 000-255 | 0-100% |
| 238 | RGB Panel 2 Green 2 fine | 000-255 | 0-100% |
| 239 | RGB Panel 2 Blue 2 | 000-255 | 0-100% |
| 240 | RGB Panel 2 Blue 2 fine | 000-255 | 0-100% |
| 241 | RGB Panel 2 Red 3 | 000-255 | 0-100% |
| 242 | RGB Panel 2 Red 3 fine | 000-255 | 0-100% |
| 243 | RGB Panel 2 Green 3 | 000-255 | 0-100% |
| 244 | RGB Panel 2 Green 3 fine | 000-255 | 0-100% |
| 245 | RGB Panel 2 Blue 3 | 000-255 | 0-100% |
| 246 | RGB Panel 2 Blue 3 fine | 000-255 | 0-100% |
| 247 | RGB Panel 2 Red 4 | 000-255 | 0-100% |
| 248 | RGB Panel 2 Red 4 fine | 000-255 | 0-100% |
| 249 | RGB Panel 2 Green 4 | 000-255 | 0-100% |
| 250 | RGB Panel 2 Green 4 fine | 000-255 | 0-100% |
| 251 | RGB Panel 2 Blue 4 | 000-255 | 0-100% |
| 252 | RGB Panel 2 Blue 4 fine | 000-255 | 0-100% |
| 253 | RGB Panel 2 Red 5 | 000-255 | 0-100% |
| 254 | RGB Panel 2 Red 5 fine | 000-255 | 0-100% |
| 255 | RGB Panel 2 Green 5 | 000-255 | 0-100% |
| 256 | RGB Panel 2 Green 5 fine | 000-255 | 0-100% |
| 257 | RGB Panel 2 Blue 5 | 000-255 | 0-100% |
| 258 | RGB Panel 2 Blue 5 fine | 000-255 | 0-100% |
| 259 | RGB Panel 2 Red 6 | 000-255 | 0-100% |
| 260 | RGB Panel 2 Red 6 fine | 000-255 | 0-100% |
| 261 | RGB Panel 2 Green 6 | 000-255 | 0-100% |
| 262 | RGB Panel 2 Green 6 fine | 000-255 | 0-100% |
| 263 | RGB Panel 2 Blue 6 | 000-255 | 0-100% |
| 264 | RGB Panel 2 Blue 6 fine | 000-255 | 0-100% |
| 265 |  | RGB Panels (1+2) Pattern Dimmer | 000-255 | 0-100% |
| 266 |  | RGB Panels (1+2) Pattern Selection | 000-001 | No function |
|  |  |  | 002-079 | One pattern per value |
|  |  |  | 080-255 | No function |
| 267 |  | RGB Panels (1+2) Pattern Selection Fade | 000-255 | Selection fade (fast->slow) |
| 268 |  | RGB Panels (1+2) Pattern Speed | 000-124 | Left to Right / Slow to fast |
|  |  |  | 125-130 | No function |
|  |  |  | 131-249 | Right to Left / Fast to slow |
|  |  |  | 250-255 | No function |
| 269 |  | RGB Panels (1+2) Pattern Fade | 000-255 | Pattern fade (fast->slow) |
| 270 |  | Reserve | 000-255 | Reserve |
| 271 |  | RGB Panels (1+2) Pattern Red | 000-255 | 0-100% |
| 272 |  | RGB Panels (1+2) Pattern Green | 000-255 | 0-100% |
| 273 |  | RGB Panels (1+2) Pattern Blue | 000-255 | 0-100% |
| 274 |  | RGB Panels (1+2) Pattern CTC | 000-255 | Pattern CTC |
| 275 | Device Settings (Hold 3s to activate) | 000-019 | No function |
|  |  | 020-024 | Display Backlight On |
|  |  | 025-029 | Display Backlight Off |
|  |  | 105-109 | Fan Auto |
|  |  | 110-114 | Fan Silent |
|  |  | 115-119 | Fan Off |
|  |  | 120-124 | Fan High Power |
|  |  | 130-134 | Invert RGB Panel 1 Off |
|  |  | 135-139 | Invert RGB Panel 1 On |
|  |  | 140-144 | Invert RGB Panel 2 Off |
|  |  | 145-149 | Invert RGB Panel 2 On |
|  |  | 150-154 | Invert Centric Tube Off |
|  |  | 155-159 | Invert Centric Tube On |
|  |  | 160-164 | Invert Centric Tube 1 Off |
|  |  | 165-169 | Invert Centric Tube 1 On |
|  |  | 170-174 | Invert Centric Tube 2 Off |
|  |  | 175-179 | Invert Centric Tube 2 On |
|  |  | 180-184 | Invert Centric Tube 3 Off |
|  |  | 185-189 | Invert Centric Tube 3 On |
|  |  | 190-194 | Invert Centric Tube 4 Off |
|  |  | 195-199 | Invert Centric Tube 4 On |
|  |  | 200-204 | Swap RGB Panels Off |
|  |  | 205-209 | Swap RGB Panels On |
|  |  | 240-244 | Factory Reset |

12. Control menu

MENU	Menu and return

DOWN	 To go backward in the selected functions

UP	To go forward in the selected functions

ENTER	To confirm the selected functions

Note:

- Screen lock password: Up Down Up Down

- Press the Enter button to enter the menu

- If there is no operation within 30 seconds, the interface will jump to the main menu

- If the current mode is DMX mode, when there is no signal input, the screen will flash after no operation for a few seconds, and will not turn off the screen

- When it’s master mode, do not enter the screen saver

Set DMX Address:

- Press "MENU" to unlock screen, then select the “DMX Address” and press "ENTER".

- Showing "Set DMX Address 001", Press the "UP or DOWN" key to increase or decrease the DMX address value.

3	Press "ENTER" to save and Exit, Press "Cancel" does not save and Exit.

| Level 1 | Level 2 | Description |
| --- | --- | --- |
| DMX Address | DMX Address | 001-512 |
| DMX Channel Mode | 15CH |  |
|  | 39CH |  |
|  | 71CH |  |
|  | 112CH |  |
|  | 209CH |  |
|  | 178CH |  |
|  | 275CH |  |
|  | 16CH |  |
| Dimming Mode | DIM1 |  |
|  | DIM2 |  |
|  | DIM3 |  |
|  | DIM4 |  |
|  | DIM5 |  |
| Mode Selection | DMX512 |  |
|  | Manual | R/G/B/W [0~255] |
|  | Auto | Auto 1~3 |
| Network Settings | Set IP address |  |
|  | Set submet mask |  |
|  | Set the Universe |  |
| System Settings | Display mode | Always On/10s Off/20s Off/30s Off |
|  | Screen Lock | On/Off |
|  | Language | Language / 语言 |
|  | No Signal | Blackout / Hold |
|  | Factory Reset | Yes / No |
|  | Fan Mode | Auto / Silent / Off / High |
|  | Screen Flip | No / Yes |
| System Information | Temperature |  |
|  | RDMUID |  |
|  | Version |  |

13. Specification

Lightsource

- Strobe Tube: 360 x 2W 6500K white leds

- Strobe Tube: 5 x 12 segments

- RGBW Panel: 432 x 2W RGBW 4in1 leds

- RGBW Panel: 2 x 6 segments

Control

- DMX Channel Modes: 15CH, 39CH, 71CH, 112CH, 209CH, 178CH, 275CH, 16CH

- Display: Illuminated graphic LCD

- Controls: 4 touch sensitive, backlighted controls

- Protocol: USITT DMX-512, RDM, ArtNet, sACN, Auto

- Cooling: Active, Forced Air, Temperature-regulated

- Thermal Protection: Automatic overtemperature protection

Connectors

- DMX I/O: Seetronic IP65 XLR 3-Pin or 5-Pin In&Out

- Power I/O: Seetronic PowerCON TRUE1 In&Out

- ArtNet I/O: Seetronic RJ45 In&Out

Operating Conditions

- Mains voltage: 100-240V AC / 50-60Hz

- Max power consumption 500W

- Temperature range, Operating: -40°C to 45°C

Mounting Options

- Mounting point by Yoke (included)

- Safety wire attachment: foldout eyelets

Shipping

Single fixture: cardboard

Tourpack: 4-way Flight Case

Housing Colors

Standard colors: black

Dimensions

Length: 420 mm

Width: 177 mm

Height (including handle): 249 mm

Weight

N.W: 10.2 kg     G.W: 11.3 kg

Packing Size

54x30x23cm