PRODUCT SPECIFICATION

MIPI 13M

Camera Module

XLS131796-V1.0

Supplier confirmation

| Department | Engineer | Quality inspector | Approver |
| --- | --- | --- | --- |
| Name / signature | [LI] | [Name / signature] | [Name / signature] |
| Date | [26-10-30] | [YYYY-MM-DD] | [YYYY-MM-DD] |

Customer confirmation

| Department | Structure / appearance | Electronics | Approver |
| --- | --- | --- | --- |
| Name / signature | [Name / signature] | [Name / signature] | [Name / signature] |
| Date | [YYYY-MM-DD] | [YYYY-MM-DD] | [YYYY-MM-DD] |

Contact information

SHENZHEN SINOSEEN TECHNOLOGY

2nd Floor, Building C, Innovation Silicon Valley Industrial Park, Jingtian Road, Guanlan Guanhu Street, Longhua District, Shenzhen, China

https://www.sinoseen.com/

Revision History

| Date | Version | Revision Details |
| --- | --- | --- |
| 2023-09-09 | V1.0 | The First Version |

Product Overview

Product Introduction

|  | XLS131796-V1.0 is a compact camera module using the S5K3L8 13MP CMOS image sensor. Featuring OmniBSI+ technology, the sensor outputs RAW images at 4208 × 3120. Image quality, data transfer and camera functions are controlled through the SCCB interface. The sensor supports up to 240 fps at full resolution. |
| --- | --- |

Applications

- Barcode scanners: Used for home surveillance, video doorbells and smart locks.

- 3D scanning and QR code scanning

- Face, gesture and iris recognition

- Industrial inspection, AR/VR headsets and pattern inspection

- Pattern inspection

- High-speed image capture

Product Specifications

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| Model | XLS131796-V1.0 | Image Sensor | S5K3L8 |
| Resolution | 4208 × 3120 (13MP) | Optical Format | 1/3.06-inch CMOS |
| Pixel Size | 1.12 μm × 1.12 μm | Shutter Type | Rolling shutter |
| Output Format | 8-bit / 10-bit RGB RAW | Dynamic Range | 67 dB (8× gain) |
| Sensitivity | 600 mV/(lux·s) | Maximum SNR | 34 dB |
| Dark Current | 8 mV/s (50°C junction) | Chief Ray Angle | 29.1° |
| AVDD (drawing) | 2.8 V | DOVDD (drawing) | 1.8 V |
| DVDD (drawing) | 1.2 V | Control Interface | SCCB |
| Standby Power | 35 µW | Active Power | 198 mW |
| Stable Image Junction Temp. | 0°C to 50°C | Operating Junction Temp. | −30°C to 70°C |
| Connector | OK-118RM024/2-35 | Input Clock | 6–27 MHz |

Lens Specifications

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| Effective Focal Length | 3.54 mm ±5% | F-number | 2.0 ±5% |
| Diagonal FOV | 78° ±3° | Lens Construction | 5P + IR |
| Distortion (drawing) | <1% | IR Filter | 650 nm |

Maximum Sensor Frame Rates

| Mode | Resolution | Maximum Frame Rate |
| --- | --- | --- |
| QSXGA | 2592 × 1944 | 30 fps |
| 1080p | 1920 × 1080 | 60 fps |
| 960p | 960p mode | 120 fps |
| 720p | 1280 × 720 | 120 fps |
| VGA | 640 × 480 | 240 fps |

These are sensor-level specifications. Junction temperatures and sensor power consumption do not represent the module’s rated ambient temperature or total system power consumption. Available frame rates depend on the module configuration and host system.

Mechanical Specifications

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| FPC Thickness | 0.15 ±0.05 mm | Shteel Stiffener | 0.2 ±0.05 mm |
| Overall Length | 17.55 ±0.2 mm | Overall Width | 10.8 ±0.2 mm |
| Overall Height | 6.2 ±0.2 mm | Base Dimensions | 8.5 × 8.5 mm (±0.2 mm) |

Product structural drawing

Connector Pin Assignment

| Pin No. | Signal | I/O | Description | Remarks |
| --- | --- | --- | --- | --- |
| 1 | MCLK | input | Clock input |  |
| 2 | SID(NC) | NC | NC |  |
| 3 | MDN0 | Output | MIPI data 0 negative electrode signal |  |
| 4 | MDP0 | Output | MIPI data 0 positive electrode signal |  |
| 5 | MDN1 | Output | MIPI data 1 negative electrode signal |  |
| 6 | MDP1 | Output | MIPI data 1 positive electrode signal |  |
| 7 | MCN | Output | MIPI clock negative electrode signal |  |
| 8 | MCP | Output | MIPI clock positive eleetrode signal |  |
| 9 | GND | Ground | Ground |  |
| 10 | RESET | Input | Reset signal input (built-up resistance, low potential) |  |
| 11 | SDA | I/O | SCCB serial interface data IO |  |
| 12 | SCL | Input | SCCB serial interface clock input |  |
| 13 | AVDD-2.8V | Power | Analog power supply |  |
| 14 | AGND | Ground | Ground |  |
| 15 | VSYNC(NC) | NC | NC |  |
| 16 | AFVDD-2.8V | Power | AF power supply |  |
| 17 | AF-GND | Ground | Ground |  |
| 18 | MDP3 | Output | MIPI data 3 positive electrode signal |  |
| 19 | MDN3 | Output | MIPI data 3 negative electrode signal |  |
| 20 | MDP2 | Output | MIPI data 2 positive electrode signal |  |
| 21 | MDN2 | Output | MIPI data 2 negative electrode signal |  |
| 22 | DGND | Ground | Ground |  |
| 23 | DOVDD-1.8V | Power | I/O power supply |  |
| 24 | DVDD-1.2V | Power | Digital power supply |  |
| 25 |  |  |  |  |
| 26 |  |  |  |  |
| 27 |  |  |  |  |
| 28 |  |  |  |  |
| 29 |  |  |  |  |
| 30 |  |  |  |  |