PRODUCT SPECIFICATION

USB 16M

Camera Module

XLS-GM974-V2.0

Supplier confirmation

| Department | Engineer | Quality inspector | Approver |
| --- | --- | --- | --- |
| Name / signature | [LI] | [Name / signature] | [Name / signature] |
| Date | [26-10-31] | [YYYY-MM-DD] | [YYYY-MM-DD] |

Customer confirmation

| Department | Structure / appearance | Electronics | Approver |
| --- | --- | --- | --- |
| Name / signature | [Name / signature] | [Name / signature] | [Name / signature] |
| Date | [YYYY-MM-DD] | [YYYY-MM-DD] | [YYYY-MM-DD] |

Contact information

SHENZHEN SINOSEEN TECHNOLOGY

2nd Floor, Building C, Innovation Silicon Valley Industrial Park, Jingtian Road, Guanlan Guanhu Street, Longhua District, Shenzhen, China

https://www.sinoseen.com/

Revision History

| Date | Version | Revision Details |
| --- | --- | --- |
| 2023-05-30 | V1.0 | The First Version |

Product overview

Product Introduction

|  | XLS-GM974-V2.0 is an 16M Ultra HD auto focus or fixed focus camera module .The maximum resolutionis 4672*3520 .Support up to 16M/30 frame high-definition picture output .The module supports USB2.0 interface,no need to install driver.The finished camera module hasthe characteristics of small size,accurate color reproduction,clear image,low power consumption andhigh reliability. |
| --- | --- |

Applications

- Video conferencing

- Security and smart home

- Document cameras

- Industrial inspection

- Identity recognition

- ATMs

- Access control equipment

Product details

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| Model | XLS-GM974-V2.0 | Interface | 4-pin, 1.25 mm pitch, USB 2.0 |
| Video output | MJPG, YUY2 | USB signaling rate | 260 Mb/s |
| Supply voltage | 5 V ±5% | Microphone | Dual digital noise-reduction microphone |
| Operating temperature | 0°C to 50°C | Storage temperature | −25°C to 70°C |
| Power consumption | No LED: / | IR LED | / |
| OS support | Windows XP / Vista / 7 / 8 / 8.1 / 10 | Other OS support | Mac / Android / Linux 2.6.2 (UVC) / WinCE |

Chip parameters

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| Sensor format | 1/2.8-inch CMOS | Effective pixels | 4672 × 3520 |
| Pixel size | 1.12 µm × 1.12 µm | Sensor data output | 10-bit RAW |
| Maximum SNR | TBD | Dynamic range | TBD |
| Minimum illumination | TBD (no LED) | Optical distortion | / |

Lens parameters

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| Lens mount | / | Construction | / |
| F-number | 2.5 ±5% | Effective focal length | 4.0 mm |
| Field of view | Diagonal: 76° ±3° | Focus distance | 60 cm to infinity |

Maximum frame rate

| Format | Resolution | Maximum frame rate |
| --- | --- | --- |
| MJPG | 2592x1944 2048x1536 1920x1200 1920x1080 1600x1200 1280x1024 1024x1024 1280x960 1280x720 1024x768 960x720 800x600 640x480 320x240 | 30 fps |
| MJPG | 4656x3496 4208x3120 | 5 fps |

Product structural drawing

USB connector pin assignment

| Pin NO | Pin Name | I/O | Description | Remarks |
| --- | --- | --- | --- | --- |
| 1 | VCC 5V | P | +5V power supply |  |
| 2 | D- | A | USB data cable- |  |
| 3 | D+ | A | USB data cable+ |  |
| 4 | GND | G | Ground |  |