PRODUCT SPECIFICATION

USB 1080P

Camera Module

SNS-GM1045-V1.1

Supplier confirmation

| Department | Engineer | Quality inspector | Approver |
| --- | --- | --- | --- |
| Name / signature | [LI] | [Name / signature] | [Name / signature] |
| Date | [26-10-23] | [YYYY-MM-DD] | [YYYY-MM-DD] |

Customer confirmation

| Department | Structure / appearance | Electronics | Approver |
| --- | --- | --- | --- |
| Name / signature | [Name / signature] | [Name / signature] | [Name / signature] |
| Date | [YYYY-MM-DD] | [YYYY-MM-DD] | [YYYY-MM-DD] |

Contact information

SHENZHEN SINOSEEN TECHNOLOGY

2nd Floor, Building C, Innovation Silicon Valley Industrial Park, Jingtian Road, Guanlan Guanhu Street, Longhua District, Shenzhen, China

https://www.sinoseen.com/

Revision History

| Date | Version | Revision Details |
| --- | --- | --- |
| 2026-01-17 | V1.1 | The First Version |

Product overview

Product Introduction

|  | SNS-GM1045-V1.1 is an 1080P Ultra HD auto focus or fixed focus camera module .The maximum resolutionis 1920*1200 .Support up to 1080P/90 frame high-definition picture output .The module supports USB2.0 interface,no need to install driver.The finished camera module hasthe characteristics of small size,accurate color reproduction,clear image,low power consumption andhigh reliability. |
| --- | --- |

Applications

- Video conferencing

- Security and smart home

- Document cameras

- Industrial inspection

- Identity recognition

- ATMs

- Access control equipment

Product details

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| Model | SNS-GM1045-V1.1 | Interface | 5-pin, 1.0 mm pitch, USB 2.0 |
| Video output | MJPG, YUY2 | USB signaling rate | 260 Mb/s |
| Supply voltage | 5 V ±5% | Microphone | Dual digital noise-reduction microphone |
| Operating temperature | 0°C to 50°C | Storage temperature | −25°C to 70°C |
| Power consumption | No LED: / | IR LED | / |
| OS support | Windows XP / Vista / 7 / 8 / 8.1 / 10 | Other OS support | Mac / Android / Linux 2.6.2 (UVC) / WinCE |

Chip parameters

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| Sensor format | 1/2.6-inch CMOS | Effective pixels | 1920 × 1200 |
| Pixel size | 3.0 µm × 3.0 µm | Sensor data output | 10-bit RAW |
| Maximum SNR | TBD | Dynamic range | TBD |
| Minimum illumination | TBD (no LED) | Optical distortion | <-8.8% |

Lens parameters

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| Lens mount | M12 × P0.25 | Construction | 2G4P |
| F-number | 2.0 ±5% | Effective focal length | 2.0 mm |
| Field of view | Diagonal: 123° ±3° | Focus distance | 60 cm to infinity |

Maximum frame rate

| Format | Resolution | Maximum frame rate |
| --- | --- | --- |
| MJPG | 1920x1200 1920x1080 1600x1200 1280x1024 1024x1024 1280x960 1280x720 1024x768 960x720 800x600 640x480 320x240 | 90 fps |

Product structural drawing

USB connector pin assignment

| Pin NO | Pin Name | I/O | Description | Remarks |
| --- | --- | --- | --- | --- |
| 1 | VCC 5V | P | +5V power supply |  |
| 2 | D- | A | USB data cable- |  |
| 3 | D+ | A | USB data cable+ |  |
| 4 | GND | G | Ground |  |
| 5 | GND | G | Ground |  |