PRODUCT SPECIFICATION

USB 1080P

Camera Module

XLS-SM987-V1.0

Supplier confirmation

| Department | Engineer | Quality inspector | Approver |
| --- | --- | --- | --- |
| Name / signature | [Li] | [Name / signature] | [Name / signature] |
| Date | [26-10-17] | [YYYY-MM-DD] | [YYYY-MM-DD] |

Customer confirmation

| Department | Structure / appearance | Electronics | Approver |
| --- | --- | --- | --- |
| Name / signature | [Name / signature] | [Name / signature] | [Name / signature] |
| Date | [YYYY-MM-DD] | [YYYY-MM-DD] | [YYYY-MM-DD] |

Contact information

SHENZHEN SINOSEEN TECHNOLOGY

2nd Floor, Building C, Innovation Silicon Valley Industrial Park, Jingtian Road, Guanlan Guanhu Street, Longhua District, Shenzhen, China

https://www.sinoseen.com/

XLS-SM987-V1.0  /  USB 1080P CAMERA MODULE

Product overview

|  | Product description USB 1080P camera module with a TBD-inch progressive-scan CMOS image sensor. The source specification lists full-resolution capture at TBD fps and applications including video conferencing, surveillance, document cameras, industrial inspection, identification, ATMs and access-control equipment. |
| --- | --- |

Specifications and features

USB 2.0 plug-and-play operation; still-image capture (JPG, BMP) and video capture (AVI, MP4) through the capture software. Exposure time, color difference, brightness, contrast and saturation adjustment. Progressive-scan imaging; no interpolation.

Product details

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| Model | XLS-SM987-V1.0 | Interface | 4-pin, 1.0 mm USB 2.0 |
| Video output | MJPG, YUY2 | USB signaling rate | 260 Mb/s |
| Supply voltage | 5 V ±5% | Microphone | Dual digital noise-reduction microphone |
| Operating temperature | 0°C to 50°C | Storage temperature | −25°C to 70°C |
| Power consumption | No LED: / | IR LED | / |
| OS support | Windows XP / Vista / 7 / 8 / 8.1 / 10 | Other OS support | Mac / Android / Linux 2.6.2 (UVC) / WinCE |

Chip parameters

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| Sensor format | TBD-inch CMOS | Effective pixels | 1920× 1080 |
| Pixel size | TBD | Sensor data output | 10-bit RAW |
| Maximum SNR | TBD | Dynamic range | TBD |
| Minimum illumination | TBD (no LED) | Optical distortion | <10% |

Lens parameters

| Item | Specification | Item | Specification |
| --- | --- | --- | --- |
| Lens mount | M6 × P0.25 | Construction | 4P |
| F-number | 2.4 ±5% | Effective focal length | 4.3 mm |
| Field of view | Diagonal: 73° ±3° | Focus distance | 60 cm to infinity |

Maximum frame rates

| Format | Resolution | Maximum frame rate |
| --- | --- | --- |
| MJPG | TBD | TBD |
| MJPG | TBD | TBD |

TBD = to be determined. “/” indicates no value provided in the source. Frame rates above are listed for MJPG; YUY2 mode details are not supplied.

XLS-SM987-V1.0  /  USB 1080P CAMERA MODULE

Product structural drawing

Mechanical dimensions, image orientation and connector pin assignment

Drawing reference

Model: XLS-SM987-V1.0   |   Units: mm   |   Drawing date: 2025-03-21

Revision markings: V1.0 / A0   |   Unspecified dimensional tolerance: ±0.1 mm

USB connector pin assignment

| Pin | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Signal | USB+5V | DM/D− | DP/D+ | GND |

The structural drawing is retained from the source specification. Refer to the dimensioned drawing for assembly and inspection.

XLS-SM987-V1.0  /  USB 1080P CAMERA MODULE

Product appearance specifications

Appearance inspection

| # | Item | Requirements | Test method | Class |
| --- | --- | --- | --- | --- |
| 1 | Module appearance | No surface dirt, no scratches visible to the naked eye, and no impurities in the plastic. | 8× magnifier | A |
| 2 | Lens | No visible scratches or dirt under 800 ±100 lux illumination, viewed head-on and with 45° reflected light. | 8× magnifier | A |
| 3 | Thread adhesive | Adhesive length: 1/3 to 1/2 of the circumference. Adhesive must not overflow onto the lens end face or lens base. | 8× magnifier | A |
| 4 | Sealant | Apply evenly between the base and PCB, without gaps or uneven thickness. No overflow beyond the FPC edge that causes the assembly dimensions to exceed the specified limits. | 8× magnifier | A |
| 5 | PCB | Surface clean; printed characters legible. Edge burr: length <1 mm, width <0.3 mm. Edge notch: length <1 mm, width <0.1 mm; no more than 2 notches per edge. | 45× microscope (continuous zoom) | A |

Dimensional inspection

| # | Item | Requirements | Test method | Class |
| --- | --- | --- | --- | --- |
| 1 | Height | Conform to the product drawing. | Digital caliper | A |
| 2 | Length | Conform to the product drawing. | Digital caliper | A |
| 3 | Width | Conform to the product drawing. | Digital caliper | A |

Defect classes A and B are defined on the following page.

XLS-SM987-V1.0  /  USB 1080P CAMERA MODULE

Product image specifications

Optical performance inspection requirements

| # | Test item | Requirements | Test method | Class |
| --- | --- | --- | --- | --- |
| 1 | Dust | No visible foreign objects anywhere in the captured white-field image. | Test fixture; small light box; 6500 K white backlight | A |
| 2 | Image orientation | Conform to the product drawing. | Test fixture | A |
| 3 | Bright and dark pixels | Dark field: <400 bright pixels; <20 two-adjacent-pixel bright clusters; no bright clusters of more than 3 pixels. White field: <400 dark pixels; <20 two-adjacent-pixel dark clusters; no dark clusters of more than 3 pixels. | Test software; small light box; 6500 K white backlight | A |
| 4 | Resolution | 1080P: center resolution ≥500 LW/PH. | MTF test procedure | A |
| 5 | FOV | Diagonal: 73° ±3°. | Test fixture; FOV reference board | A |
| 6 | Shading | At the four corner positions described as “90% away from the center”: mean corner brightness ≥45% of center brightness; each corner >35% of center. Sampling window: 40 × 40 pixels; sensor vignetting compensation disabled. | Test software; small light box; 3400 lux, 6500 K white backlight | A |
| 7 | Color | Color reproduction error ≤20%. | Test software; color checker | A |
| 8 | TV distortion | Pincushion (+) or barrel (−) distortion: no more than 1%. | Test software; ISO 12233 chart | A |
| 9 | Grayscale | Brightness difference between adjacent gray levels <8. | Test software; grayscale chart | A |
| 10 | CA | <0.5. | Test software; ISO 12233 chart | A |
| 11 | Torque | ≥0.5 kgf·cm. | Torque wrench | A |
| 12 | Thrust | ≥2 N. | Thrust gauge | A |

Defect classification

A — Major nonconformity: affects image quality, principal functions or performance specifications, or results in rejection of the inspected batch.

B — General nonconformity: does not affect final product appearance, technical performance, or subsequent processing and assembly.

Source criteria retained. CA units, grayscale calculation, color-error method and the exact shading measurement positions require engineering confirmation before customer approval.

XLS-SM987-V1.0  /  USB 1080P CAMERA MODULE

Product sampling plan

Imaging and reliability

| # | Test item | Sampling plan | Test method / remarks |
| --- | --- | --- | --- |
| 1 | Dust | Each batch: N=5, C=0 | Same as production; production inspection |
| 2 | Image orientation | Each batch: N=5, C=0 | Test fixture; production inspection |
| 3 | Bright and dark pixels | Each batch: N=5, C=0 | Same as production; production inspection |
| 4 | Resolution | Each batch: N=5, C=0 | MTF test procedure; production inspection |
| 5 | FOV | Each batch: N=5, C=0 | Test fixture / FOV board; QA sampling and full evaluation at sample stage |
| 6 | Image uniformity | Each batch: N=5, C=0 | Test software; 3400 lux, 6500 K light box; production inspection |
| 7 | Dispersion | Each batch: N=5, C=0 | Test software / color checker; QA sampling and full evaluation at sample stage |
| 8 | TV distortion | Each batch: N=5, C=0 | Test software / ISO 12233 chart; QA sampling and full evaluation at sample stage |
| 9 | Grayscale | Each batch: N=5, C=0 | Test software / grayscale chart; QA sampling and full evaluation at sample stage |
| 10 | Reliability | Trial production: 2 pcs / production batch. Stable production: 5 pcs / test item / month. | Acceptance level: 0/1, as stated in the source. |

Appearance

| # | Test item | Sampling plan | Test method / remarks |
| --- | --- | --- | --- |
| 1 | Module appearance | AQL=1.0 | Visual; production inspection |
| 2 | Lens | AQL=1.0 | 8× magnifier; production inspection |
| 3 | Thread adhesive | AQL=1.0 | 8× magnifier; production inspection |
| 4 | Sealant | AQL=1.0 | 8× magnifier; production inspection |
| 5 | FPC | AQL=1.0 | Visual; production inspection |
| 6 | Gold fingers | AQL=1.0 | Visual; production inspection |
| 7 | Steel stiffener | AQL=1.0 | Visual; production inspection |
| 8 | Double-sided tape | AQL=1.0 | Visual; production inspection |

Dimensions and mechanical checks

| # | Test item | Sampling plan | Test method / remarks |
| --- | --- | --- | --- |
| 1 | Height | AQL=1.0 | Same as production; production inspection |
| 2 | Length | AQL=1.0 | Same as production; production inspection |
| 3 | Width | AQL=1.0 | Same as production; production inspection |
| 4 | Thread torque | Each batch: N=5, C=0 | Torque wrench; QA sampling |
| 5 | Base thrust | Each batch: N=5, C=0 | Thrust gauge; QA sampling |

N = sample size; C = acceptance number. AQL values are reproduced from the source; the governing sampling standard and inspection level are not specified.

XLS-SM987-V1.0  /  USB 1080P CAMERA MODULE

Product reliability specifications

| # | Test item | Conditions and requirements | Test equipment |
| --- | --- | --- | --- |
| 1 | Low-temperature storage | −30°C for 48 h. Return to room temperature for 2 h before testing. | Low-temperature chamber + computer |
| 2 | Low-temperature operation | −20°C; operate the product and inspect the image for 48 h. | Low-temperature chamber + computer |
| 3 | High-temperature storage | 80°C for 48 h. Return to room temperature for 2 h before testing. | High-temperature chamber + computer |
| 4 | High-temperature operation | 70°C; operate the product and inspect the image for 48 h. | High-temperature chamber + computer |
| 5 | Aging | Operate at room temperature for 36 h and check for image abnormalities. | Computer |
| 6 | Connector mating | Connector plugging / unplugging. Cycle count and detailed conditions are not specified in the source. | Workbench + computer |
| 7 | Electrostatic discharge | Apply ±6 kV contact discharge and ±8 kV air discharge at the exposed copper grounding location. Apply 10 discharges per polarity at each point, with a 1 s interval. | ESD equipment + computer |
| 8 | Free drop after packaging | Drop the packaged product freely from 150 cm, then check the image. | Oak floor + computer |
| 9 | Vibration after packaging | Fix the packaged product to the test bench. Sweep 10–100–10 Hz; amplitude 2 mm; X, Y and Z directions; test duration 2 h as stated in the source. Check the image after testing. | Vibration test bench + computer |

Acceptance requirements

After the reliability tests, dimensions, appearance, torque and optical performance must conform to the product requirements.

Post-test inspection conditions

| Item | Condition |
| --- | --- |
| Small light box | 1500 lux ±10% |
| Large light box | 6000 lux ±10% |
| Color temperature | 4400 K ±150 K |
| Inspection system | PC test software / module test platform |

Reliability test conditions are reproduced from the source and do not extend the rated operating or storage ranges on page 2. This page defines test requirements, not a completed test report.