ERROR CODES LIST

Pro Max / Pro Max Alpine Smart Controller

| Code | Fault | Controller meaning | Required checks |
| --- | --- | --- | --- |
| E-01 | Start-up failure | Two ignition attempts failed. | Check fuel supply, fuel-line priming, combustion-air intake and exhaust path. Correct the cause before restarting. |
| E-02 | Flame-out | The flame went out during operation. | Check fuel level, dosing-pump operation, fuel-line connections and air bubbles in the fuel circuit. |
| E-03 | High voltage | Supply voltage remained above 16 V for 5 seconds. | Check charger/alternator output, power supply and system voltage. Do not operate until voltage is within the correct range. |
| E-04 | Low voltage | Supply voltage remained below 10 V for 20 seconds. | Charge/test the battery; inspect cable size, terminals, fuses and voltage drop during start-up. |
| E-05 | Air-outlet temperature sensor fault | Sensor circuit is open or short-circuited. | Inspect the sensor connector and harness. Electrical testing or sensor replacement should be performed by qualified service personnel. |
| E-07 | Fuel-pump fault | Fuel-pump circuit is open or short-circuited. | Inspect the pump connector, harness and pump coil. Do not bypass the control circuit. |
| E-08 | Fan fault | Fan circuit is open or short-circuited. | Check for a blocked fan, damaged connector or wiring fault. Internal fan/ECU service requires qualified personnel. |
| E-09 | Air-outlet overtemperature | Outlet-temperature sensor detected more than 160 °C. | Switch off and allow the heater to cool. Check hot-air ducts, outlet, intake and safety grids for restriction or recirculation. |
| E-10 | Glow-plug fault | Glow-plug circuit is open or short-circuited. | Inspect the glow-plug connector and harness. Test or replace the glow plug through qualified service. |
| E-11 | Heater-body overtemperature | Aluminium-body temperature exceeded 260 °C. | Switch off and cool the heater. Check whether the hot-air outlet or duct is blocked and verify adequate airflow. |
| E-12 | Overtemperature-sensor fault | Overtemperature-sensor circuit is open or short-circuited. | Inspect the sensor connector and harness. Qualified electrical diagnosis is required. |

Clearing a Fault Display

| 1 | Stop the heater and correct the cause of the fault. |
| --- | --- |

| 2 | With the heater in standby, press and hold the Power/OK key to clear the displayed code. |
| --- | --- |

| 3 | Press Power/OK to start the heater only after the fault has been eliminated. |
| --- | --- |

| 4 | If the fault remains, the code will reappear. Do not continue resetting the controller; arrange qualified service. |
| --- | --- |

Safety: Do not restart after E-09 or E-11 until the heater has cooled and the airflow restriction has been removed. Do not operate the heater with a fuel leak, exhaust leak or unresolved electrical fault.

Basic Checks Before Service

| Check | What to confirm |
| --- | --- |
| Power supply | Correct 12 V or 24 V system; battery charged; terminals tight; fuses correct; no excessive voltage drop. |
| Fuel system | Correct fuel; adequate fuel level; no leaks, kinks or bubbles; pump installed at the correct angle. |
| Combustion air | Intake clear of snow, dirt, water and exhaust fumes. |
| Exhaust | Outlet clear, routed outdoors and not kinked or blocked. |
| Heater air | Intake, ducts, outlet and safety grids unobstructed; hot air is not immediately recirculated. |
| Connectors | Controller, pump, fan, sensors and glow-plug connectors fully seated and undamaged. |