# Smart Controller Manual

Pro Max (Brushless) Air Heater Controller

Version 1.0

For Pro Max / Pro Max Alpine air heaters

# Contents

| 1 | Controller Overview |
| --- | --- |
| 2 | First Power-Up and Main Interface |
| 3 | Heating Operation |
| 4 | Ventilation Mode |
| 5 | Manual/Thermostat Mode |
| 6 | Remote Control Pairing |
| 7 | Fuel Priming |
| 8 | Time and Date |
| 9 | Timer Programs P1-P3 |
| 10 | Status Display |
| 11 | Screen-Off State |
| 12 | Safe Operation Notes |

# 1  Controller Overview

Smart Controller key layout

| Key | Main functions |
| --- | --- |
| Left key | Select the previous menu item. During heating, increase the level or target temperature. During setup, adjust the flashing value as indicated on screen. |
| Right key | Select the next menu item. During heating, decrease the level or target temperature. During ventilation time setup, increase the running time; use the on-screen value to confirm direction in other setup menus. |
| Return key | Return to the previous menu. Press and hold for 3 seconds during operation to switch between manual output mode and thermostat mode. |
| Power/OK key | Confirm a selection, enter a menu and start a function. Press and hold for approximately 3 seconds to stop heating or ventilation. |

# 2  First Power-Up and Main Interface

| 1 | Connect power to the heater and controller. |
| --- | --- |

| 2 | The screen opens at the time-setting interface. Press Power/OK to confirm or enter the week setting. |
| --- | --- |

| 3 | Press Power/OK again to open the main interface. |
| --- | --- |

| 4 | Use the Left and Right keys to move between Heating, Ventilation, Timer and Settings. |
| --- | --- |

| 5 | When the required icon flashes, press Power/OK to enter that function. |
| --- | --- |

Standby shortcut: From the main interface in standby, short-press Power/OK to enter the heating run-time setting. Press Power/OK again to start heating.

# 3  Heating Operation

## 3.1 Start heating

| 1 | Select the heating icon and press Power/OK. |
| --- | --- |

| 2 | Set the operating time with the Left/Right keys. The selectable range is 10-120 minutes. After 120 minutes, the next setting is continuous operation (∞). |
| --- | --- |

| 3 | Press Power/OK to start heating. |
| --- | --- |

| 4 | Use the Left key to increase and the Right key to decrease the manual output level or thermostat temperature. |
| --- | --- |

## 3.2 Stop heating

| 1 | Press and hold Power/OK for approximately 3 seconds. |
| --- | --- |

| 2 | The display shows OFF and the heater begins its automatic shutdown and cool-down cycle. |
| --- | --- |

| 3 | Wait until the heater has completely stopped before disconnecting the power supply. |
| --- | --- |

After-run protection: Do not use a battery isolator or disconnect the supply during normal shutdown. Interrupting the after-run cycle can damage the heater.

# 4  Ventilation Mode

| 1 | On the main interface, select the FAN icon and press Power/OK. |
| --- | --- |

| 2 | Set the running time. In the current controller instructions, the Left key reduces the time and the Right key increases it. |
| --- | --- |

| 3 | Press Power/OK to begin ventilation. |
| --- | --- |

| 4 | Adjust fan speed from L1 to L10 with the Left and Right keys. |
| --- | --- |

| 5 | To stop ventilation, press and hold Power/OK for approximately 3 seconds. The controller returns to the main interface. |
| --- | --- |

# 5  Manual/Thermostat Mode

| Operating state | Action | Display result |
| --- | --- | --- |
| Heater running | Press and hold Return for 3 seconds. | Thermostat mode is active when a target temperature such as 30 °C appears in the upper-right corner. |
| Thermostat mode active | Press and hold Return for 3 seconds again. | Manual mode is active when a level such as L4 is displayed. Manual mode is the default. |
| Using remote control | Press and hold the remote-control power key for 3 seconds. | The controller cycles between manual and thermostat mode. Wait more than 3 seconds before switching again. |

# 6  Remote Control Pairing

| 1 | Place the heater in standby. |
| --- | --- |

| 2 | Press and hold the Left key and Return key together for 3 seconds to open the remote-pairing interface. |
| --- | --- |

| 3 | Press and hold the power key on the remote control. |
| --- | --- |

| 4 | After successful pairing, the controller exits the pairing interface automatically. |
| --- | --- |

| 5 | To cancel, short-press Power/OK. The controller also exits automatically after approximately 20 seconds without input. |
| --- | --- |

One Smart Controller can be paired with up to two remote controls.

# 7  Fuel Priming

Use only when required: Priming is intended for the initial fuel-line fill after the power supply is connected. Excessive priming can introduce too much fuel into the heater.

| 1 | With the heater in standby, press and hold the Right key for 3 seconds. |
| --- | --- |

| 2 | When the display shows Priming, press Power/OK to start. |
| --- | --- |

| 3 | After successful ignition, the heater enters normal operation. |
| --- | --- |

| 4 | Priming can be used only once after the first power connection. To use it again, disconnect and reconnect the power supply. |
| --- | --- |

# 8  Time and Date

| 1 | In standby, select the Settings icon with the Left/Right keys and press Power/OK. |
| --- | --- |

| 2 | Select the flashing clock icon and press Power/OK. |
| --- | --- |

| 3 | Set the hour in 24-hour format with the Left/Right keys, then press Power/OK. |
| --- | --- |

| 4 | Set the minutes and press Power/OK. |
| --- | --- |

| 5 | Select the day of the week: MON, TUE, WED, THU, FRI, SAT or SUN. |
| --- | --- |

| 6 | Press Return to go back, or Power/OK to return to the main interface. |
| --- | --- |

# 9  Timer Programs P1-P3

The controller provides three programmable start times. P1, P2 and P3 may be set for the same day or for different days.

| 1 | On the main interface, select the P timer icon and press Power/OK. |
| --- | --- |

| 2 | Select P1, P2 or P3, then press Power/OK. |
| --- | --- |

| 3 | Select the day of the week and press Power/OK. |
| --- | --- |

| 4 | Set the start hour and minute, confirming each field with Power/OK. |
| --- | --- |

| 5 | Set the running time from 10-120 minutes or continuous operation (∞). |
| --- | --- |

| 6 | Select ON or OFF with the Left/Right keys and confirm. |
| --- | --- |

| 7 | Press Return to return to the main interface. |
| --- | --- |

Timer status: When a timer is enabled, the timer icon appears and the active program is marked. After a programmed operation finishes, the check mark for that program is cleared.

# 10  Status Display

| 1 | While the heater is running, press and hold the Left and Right keys together. |
| --- | --- |

| 2 | The status interface displays power-supply voltage, motor speed (RPM), fuel-pump frequency and Alpine elevation data. |
| --- | --- |

| 3 | Use the Left or Right key to change pages. |
| --- | --- |

# 11  Screen-Off State

The screen turns off after approximately 3 minutes without key operation, both in standby and during operation. If an indicator is blinking while the screen is off, press any key to wake the controller.

# 12  Safe Operation Notes

After a long period of non-use, check that the heater and accessories are securely mounted.

Inspect the fuel system visually for leaks and use fresh fuel where possible.

Pro Max 2 kW Alpine heaters adjust automatically from sea level to 3,000 m (9,842 ft).

Pro Max 5 kW Alpine heaters adjust automatically from sea level to 5,500 m (18,044 ft).

No owner altitude adjustment is required on Alpine versions.

When a fault code appears, correct the fault before clearing the display. Refer to the separate Error Codes List.